Installation and Operation Instructions

for the PacComm TNC for the TAPR/AMSAT DSP-93     as of 11/25/95

Background

This TNC for the DSP-93 is a 'piggyback' project on a PacComm commercial AVL

modem development. By making a few modifications to the AVL design, the board

serves as an 'inside' TNC for the DSP-93.

Various components are not mounted on the circuit board. These components (and

those on the extra inch of circuit board which has been severed) are part of

the AVL modem. Missing parts do not indicate any incompleteness of your unit.

Initial units are being shipped with an interim EPROM. This EPROM does not

contain KISS. Fully capable EPROMs will be provided at no charge when

available. See the support paragraph at the end of this instruction sheet.

Preparation

You should decide a few things about how you want to install the TNC before

you start.

LEDs

The TNC LEDs may extend through the front panel if you care to drill the

appropriate holes. The LEDs are positioned so that they do not interfere with

any of the markings screened on the front panel.

If you do not wish to drill the front panel, simply bend the LEDs upright.

DSP Load Function

The DSP load function lets you use the TNC's serial port to load DSP code as

well as talk to the TNC instead of loading the DSP via the 9 pin serial port

on the DSP-93.  This is accomplished by wiring the TNC's second serial port

(at TTL level) into the socket of the DSP-93 serial port. Note that you must

choose to do one or the other i.e. attaching the TNC connector makes the DSP-

93 serial port inoperative.

If you choose not to use the DSP Load Function, remove the header plug and

wires which are wired to the bottom of the TNC board by unsoldering or

clipping the wires.  Or  you may wrap the header plug in insulating material

and let it remain unconnected between the TNC and DSP-93 board.

Power

If you choose to power the TNC independently rather than from the DSP-93 AC

supply, you may do so by attaching  wires carrying either AC or DC voltage to

the circuit board by the rear panel of the DSP-93. Note, however, that the TNC

will need to be externally switched since no power switch is fitted. A TNC-2

style power switch may be mounted at S1, however the push button will protrude

through the AMSAT logo.

The alternate power connection holes are labeled. The center hole, AC/DC1 is

common for either type of supply voltage. Use AC2 or DC1 as appropriate for

the voltage supplied. Be sure to remove the pins from J205 so that the TNC

power rails are isolated from the DSP-93 supply.

Installation

1. Detach the ribbon cable connected to J3 on the top board of the DSP-93.

2. Remove the four screws holding the front panel and the four screws holding

the rear panel of the DSP-93 case. Let the front and rear panels move away

from the case frame to allow working room.

3. Remove the four screws holding the top DSP-93 board in place. Save these

screws for re-use.

4. Install a spacer (provided) at each corner of the top DSP board. Screw the

threaded end of the spacer into the spacer below the DSP board and tighten. Do

not overtighten as it is easy to strip the spacer threads.

5. If you are not going to use the DSP load function, proceed to step 6.

(Ignore step 5 until EPROM supports this function.)

a. Remove the MC145406 IC from U21 and put it away for future use.

b. Insert the header plug which is wired to the bottom of the TNC into the

socket at U21.

6. Align the pins of J205 and the J3 socket (20 pin socket) respectively with

JP205 and the 20 pin modem connector on the top DSP-93 board. Press the TNC

squarely down until the four corners of the TNC rest firmly on the corner

spacers.

7. Install the four screws saved in step 3 into the four new spacers to hold

the TNC in place.

8. Plug the ribbon cable connector coming from the rear panel DB-25 onto the

pins at J1 on the TNC board. Pin 1 to pin 1, etc. Pin one is the pin nearest

C4. (Leave the ribbon cable from the DB-25 flat and fold it over to plug into

J1).

9. Hold the rear panel in place and replace the four holding screws. If

necessary, rotate the speaker socket so that it clears the TNC board.

10. If you plan to drill the front panel for the TNC LEDs, perform the

following steps. If not, proceed to step 10.

a. Remove the potentiometer and reset switch so that the front panel may be

removed to the drilling area.

b. Place masking tape over the area were the drilling is to be done and press

it firmly in place. This will help reduce the chance of marring the panel.

c. Tape the drilling template to the front of the panel carefully aligning the

LED and mounting holes. Center punch the 5 LED holes (or some lesser number of

holes if you don't want them all to be showing).  It is not recommended that

you try to drill without first deeply center punching the hole location.

d. Drill each location from front to back with a sharp 3/16" drill and

carefully debur. Remove the tape.

e. Remount the potentiometer and reset switch, positioning the potentiometer

as described in the next step.

11. Loosen the mounting nut on the potentiometer at the upper left side of the

front panel and rotate it so that the wiring harness attachment points are on

the inboard side of the unit, not on the bottom side. This allows clearance

for the TNC board.

12. Hold the front panel in place, aligning it for the LEDs to protrude (if

drilled for same) and replace the four holding screws.

This completes the installation.

Jumpers

There are four jumper selections which may be made. The jumpers are located

along the edge of the circuit board just behind the reset switch on the front

panel. Each of the four jumpers are configured the same way: if the two

frontmost pins are jumpered the signal is pulled low; if the two rearmost pins

are jumpered the signal is pulled high.

The jumpers are labeled: FREQ1, FREQ2, PTT1, PTT2.

These were previously accessed via the DB-25 connector.

Operation

Refer to the Operating Manual and Errata sheet for  firmware commands and

operating instructions. PacComm firmware is derived from TAPR and we have

maintained a strong similarity in the command structure. Use of the DSP-93 TNC

should appear very similar to operating a TNC-2.

Baud Rates

The data rates on the three TNC ports are set by the BAUD command.

The terminal defaults to 9600, 8, N, 1.

Pass Through Function

If the connection was made to the DSP-93 at U21, the TNC's serial port may be

tied to the DSP serial port through use of the PASSTHRU mode. This will allow

loading the DSP code via the TNC's serial port, and allow DSP output which is

not HDLC to be passed unchanged to the terminal.

The PASSTHRU mode is entered by giving the command PASSThru ON.  When ON, all

bits entering the TNC serial port are transferred out the second serial port

(DSP-93 load port) and vice versa. When PASSTHRU is OFF the second serial port

is ignored.

PASSTHRU is a fully transparent mode. Any bit pattern may be sent either

direction without causing any effect on the TNC's command state or operation.

To get out of PASSTHRU mode, use the same scheme as is used for exiting

Transparent mode, e.g. one second of silence, three CTRL-C within one second,

and one additional second of silence.

Other Features

Parallel I/O

Eight parallel I/O lines are available at J4 for user-specific applications.

These lines are controlled via the IN and OUT commands. Connection to J4 is

left to the user. It is recommended that opto-isolators or other buffering be

provided to prevent damage to the processor I/O lines. If such damage is

sustained, the entire processor IC must be replaced.

Care should be taken in using the IN and OUT commands, especially the OUT

command. If one of the processor internal registers is accidentally specified

it is possible to crash the firmware. This may require a hard reset and all

parameters will be lost.

Analog-to-Digital Converter

The circuit board is wired to accept a PacComm AD-4/Header analog-to- digital

converter board at J2 as part of the AVL project.  There is no support for the

A-D converter in the EPROM at this time.  Contact PacComm if you have further

interest in this.

Notes

Heat -Stroke on the DSP-93. Some DSP-93 owners have reported their units

suffering from heat related problems. These problems have been analyzed by

TAPR and solutions found. Adding the TNC into the case should not cause a heat

or power problem.

PacComm Operating Manual Errata and Updates, November 27, 1995

The latest version of the Operating Manual Operating Manual for PacComm Packet

Controllers, 6th Ed., 1st Printing, May, 1994.

The TNC for the DSP-93 version of PacComm packet firmware is labeled "Release

32.2.9.2 11/25/95."

The BAUD command is somewhat different than shown in the Operating Manual.

See explanation in the PicoPacket Technical Manual.  The GPS baud rate setting

applies to the optional Second Serial Port.

KISS mode is not yet properly implemented. Do not use the KISS command as this

will cause the TNC to lock up and all parameters will be lost.

The CAL function (tune) does not work correctly.

RAM greater than 32k is not yet supported.

The TNC will not do a digital or analog loopback. Test only by connecting to

or via another packet controller.

New features in Release 3.2.9.1 and later releases not covered in the manual:

a) The monitor command remains operative when GPS is set to ON, i.e. the

monitor function is on or offdepending on the setting of MON regardless of

whether GPS is on or off regardless of the setting of GPS.

b)The MYROSE parameter holds the callsign of the local ROSE switch.  Then a

connect command to the ROSE network may be made in the form:

C Call @ Destination Switch

The connect request will be properly reformatted and sent to the local switch.

The following new commands are in this pre-release of 3.2.9.2 code:

MY1alias, MY2alias, MY3alias are additional callsign alias fields. MYAlias

remains unchanged.

PRefix specifies a character which will be prefixed to the beginning of each

'monitored' frame. It is used by some type of terminal programs to segregate

monitored frames from connected data. The default value is $00 which adds no

character to each frame.

ELOc (Echo Location) is a PicoPacket command which was included by mistake. It

should be left OFF.

EPROM Changes Planned

Subsequent EPROM releases will have a revised BAUD command menu offering rates

between 300 and 19,200 on the HDLC and Terminal ports.

The Default baud rates will be set to 19,200.

The KISS function will be implemented.

Suggestions are welcomed.

Support

You may obtain support for this product via any of the following methods:

Email to the TAPR DSP-93 SIG. PacComm monitors this SIG continuously. Contact

TAPR for instructions on subscribing to this SIG.

Future EPROM releases will be available in the TAPR DSP-93 firmware library or

via direct from PacComm.

Email direct: dsptnc@paccomm.com.

Compuserve: 76576,2003.

PacComm BBS: +(813) 874-3078.

PacComm facsimile: +(813) 872-8696.

PacComm voice technical support: +(813) 874-2980.

PacComm Packet Radio Systems, Inc.

4413 N. Hesperides St., Tampa, FL 33614-7618 USA

More information about the DSP-93 project may be obtained by ftp from

tapr.org. Use anonymous login and use your IP address as the password.