From Lon Cecil  WB5PKJ   cecil @ rfm.com

Provide a baseline set of hardware test routines for the DSP93

boards.  Hopefully, the operation and  explainations will be

useful and require little support or training.

TESTLEDS

This is the first routine to run, after initial power up of

the DSP-93 system.  At initial power up, LED 218 should

illuminate from the +5 VDC power supply.  After the initial

CPU reset time out, the TAPR DSP monitor turns on LED 211

(the left-most LED) and waits for a program to be downloaded

via the serial RS-232 port.  TESTLEDS can be downloaded to

flash the LEDs in sequence, to assure general operation of

the analog board and the digital interface to the CPU board.

After the short load delay (about 4 seconds, maximum), all

LEDs except the power LED will go dark for about 2 seconds.

Starting with LED 1 on the left, each LED will light for

about 2 seconds in sequence, moving to LED 7, near the power

LED.  Any LED that remains dark may be broken,  installed

with reversed polarity, or have a broken limiting resistor

near U215.  If  LEDs 5, 6, or 7 light at the same time

(ignore the power LED), then carefully review solder joints

near U213, U214, and U215 for bridges.  Check U206 and U215

for errors in LED 3 or 4, and U204 and U215 for LED 1 or 2.

If U215 is removed from the socket, a test lead (with 100 ohm

series resistor soldered to the end) can be used to ground

output pins 10 through 16 for manual tests.    Use the

digital ground connection near J203.

TESTRAMP

This program can be downloaded to provide a low frequency

stairstep test signal.  The signal goes from +3 to -3 VDC,

and uses selected digital voltage levels.  For easy

triggering, an oscilloscope can be connected to J203 pin 10

to monitor the output voltage level.  After a brief flash

sequence of the LEDs to show program load, the stairstep

ramp signal will begin.  LED 1 thru 7 will be dark, when the

waveform begins.  If a speaker is connected, the output is

at peak possible amplitude and will be a very loud clicking

noise.

CAUTION: this signal is applied to Radio Port 1 & 2.

Disconnect the radio interface or connect a dummy load

before running this program.   Port Control lines are reset

to high state (+5 VDC), which may tune some radios (Yaesu

FT736R, for example).

There is a short positive pulse at the front of the

waveform, to trigger the scope,  Use 1 volt per division

vertical sensitivity and 10 milliseconds horizontal sweep,

with positive edge (AC or DC) triggering.  The stairstep

should decend from +3 volts to 2.2 volts, then 1.1,  0.5,

-0.5, -1.1, -1.8, -2.3, and then -3.0 volts.  After the -3.0

volt step, the waveform returns to 0.0 volts, until the next

start pulse.  The signal repeats until the CPU is reset.

There may be significant droop to each step, due to the

limited low frequency response of the D/A converter and

amplifier system.  Since this signal is generated in the AIO

chip U208, none of the A/D and input multiplexer switches

are involved.  If the signal is unipolar (does not have a

negative voltage step), check the wiring for the  negative 5

VDC supply.

Each voltage step is directly loaded in the source code, and

the user can easily change the levels to generate any

arbitrary waveform, within the limits of the TAPR DSP

system.  This code can be the basis of  an RY generator,

audio code generator,  or packet flag simulator, for

example.

TESTBUZZ

This program can be downloaded to provide an audio test

signal.  The signal is similar to the TESTRAMP signal,

except for a reduced start pulse and a 4 millisecond total

period.  This signal is representative of the typical

communication signal needs and frequency responses, with

energy bands from 250 Hz  to well over 5 kHz.

CAUTION: this signal is applied to Radio Port 1 & 2.

Disconnect the radio interface or connect a dummy load

before running this program.  Port Control lines are reset

to high state (+5 VDC), which may tune some radios (Yaesu

FT736R, for example).

After a brief  sequence flash of the LEDs, LED 2 will remain

lighted to indicate CPU active (not reset) state and

termination of downloads until CPU is manually reset.   The

start pulse is reduced to 1/2 amplitude and appears as a

step on the rise of the main waveform, but the other voltage

information and troubleshooting information is listed in

TESTRAMP.

TESTGAIN

This test uses an oscilloscope to measure the gain for each

step of the Gain Multiplexer circuit U201.  The test signal

from an external generator or radio is connected to Radio

Port 1 Pin 8 Receive.  The monitor point is Pin 7 of U202.

Peak signal is +/-5 volts at this test point.

CAUTION: Control signals are applied to Radio Port 1 & 2.

Disconnect the radio tuning cable wires or connect a dummy

load before running this program.  Port Control lines are

reset to high state (+5 VDC), which may tune some radios.

LED 4 lights to indicate complete download and operating

program.  LEDs 5, 6, and 7 are used to indicate the binary

weight of the gain, determined by R201 and R203 thru R208.

The gain with LED 5 lighted is approximately normal

operating gain of 16X.  Note that the setting of R215 input

level control will affect the measured absolute gain.

Program loops at each gain setting for 5 seconds, then steps

to the next higher level.  With LED 5, 6, and 7 lighted, the

amplifier is in the comparator mode, and should be clipped

at both peaks.  Program runs until CPU is manually reset.

This program could be modified to activate the A/D

converter and feed a copy of the signal to the speaker, to

eliminate the need for internal probing.  A version to use

Port 1 Aux In (pin 7) would eliminate the input level pot

variation.  Suggested names TESTREC1 and  TESTAUX1,

respectively.

TESTANIN

The Port 1 Auxillary Input (Pin 3 AUX) is connected to the

gain selector and presented to Pin 7 of U202 for

verification with an oscilloscope.  The AUX input does not

have gain control or DC blocking, so this program can be

used during scope probing of U203 and U202.

CAUTION: Control signals are applied to Radio Port 1 & 2.

Disconnect the radio interface or connect a dummy load

before running this program.  Port Control lines are reset

to high state (+5 VDC), which may tune some radios.

This routine is a variation of TESTGAIN, modified for a

single input and a single gain setting.  This code is a

general basis for any fixed test code you may need to hack

for analog testing. This program could be modified to

activate the A/D converter and feed a copy of the signal to

the speaker, to eliminate the need for internal probing and

to act a signal monitor routine for cable tests.

This program could be modified to activate the A/D

converter and feed a copy of the signal to the speaker, to

eliminate the need for internal probing.

TESTPORT

This routine tests the Push to Talk (PTT) and Frequency Up

(FU) and Frequency Down (FD) lines of  Radio Ports 1 and 2.

As compiled, the code sets the ports to match the common

configuration, in which each control is active low (*) for

all controls (PTT*/FD*/FU*)  Each control line of each port

is toggled active for approximately 5 seconds, with a

corresponding front panel light.  The code repeats until the

CPU is manually reset.

CAUTION: Control signals are applied to Radio Port 1 & 2.

Disconnect the radio interface or connect a dummy load

before running this program.  Port Control lines are

changed, which may tune some radios or cause undesired

keying of the transmitter.

Radio Port 1       Radio Port 2

PTT                     LED 7               LED 4

FU                      LED 6               LED 3

FD/ICOM                 LED 5               LED 2

The files TESTPORY.ASM and TESTPORY.OBJ are a version of TESTPORT

modified  to work with radios that are active low for PTT, but

active high for other lines (PTT*/FD/FU), such as the Yaesu 736R."

ALLMONL1

ALLMONL2

ALLMONH1

ALLMONH2

This group of program codes are included as a monitor driver

for the connected radio, much as the TNC-2 has an allpass or

monitor mode for digital to transmissions.  The L# or H#

indicates the tune line mode active Low (FD*) or active High

(FD).  The digit indicates the Radio Port that is the audio

source.  LED 2 is used to indicate the program is active.

By selection of the correct version of this program, the

user should be able to directly connect a radio to the TAPR

DSP and not have to disable tune and PTT functions.