MEASGAIN.ASM Version 1.2 August 24, 1994

Program to measure the gain with the software selectable gain settings.

To assemble, use the command "asm measgain.asm -dTONES" to produce

audio loop-back or "asm measgain.asm -dINPUT" to measure gain from

input on radio 1

=============

MEASGAIP.OBJ

=============

Input mode (-dINPUT):

Sweeps the audio input from radio port 1 through each gain setting and

prints a value to the terminal.

============

MEASGATN.OBJ

============

Audio loop-back mode (-dTONES):

Generates a tone internally and sweeps the audio input through each

gain setting and prints a value to the terminal.

Loops this audio output back to radio port 1 (unplug the cable from port 1).

Steps the audio output/input frequency, prints the frequency,

then steps through each gain setting.

Both modes:

For each step, the program prints the step number and the mean square

of the 128 samples at each step. Uses the upper 10 bits of each sample.

The samples at each step are averaged and adjusted to zero

average before the mean square is calculated.

When the program starts, LED 211 is turned off.  When the program is

running, LED 215 (MSB), LED 216, and LED 217 (LSB) indicate the

gain step.

If the sampling rate is too fast for your input signal, adjust CMDA

and CMDB

The program waits for a character to be typed from the keyboard before

proceeding to allow time to set up a terminal program to observe the

mean square numbers.

LED 214 lights and LED 217, LED 216, and LED 215 begin to cycle.

The program runs until the DSP-93 is reset.

Ron Parsons W5RKN