-----------------------------------------------------------

*** We  are  going  through  the  following  tests  on  any

release.

Default bbs memory  allocation  is  abt  251K.  We  have

worked with this amount of memory without problems,with the

following configuration:

1) 8 tnc channels total.4 on the 1st port and 4 on the 2nd

port. (Sysop's channel and  dos  task  are  always  defined

internal. Each additional channel,allocates  about  11K  of

memory.)

2) 1000 msgs active,2000 bids active.

3) Maximum continuous duty,8 days.(Without unload or power

off.We had to switch off the system on the 9th day to avoid

overheating,hi)

-----

We have worked bbs with a total of 20 channels including

Sysop. The whole memory allocation  adjusted  to  380K.  We

used pk232 on the 1st port and tnc-2 on the 2nd port with

NordLink 18 channels firmware. We had 8 channels  on  pk232

and 10 channels on tnc-2. The   result   of    this    test

was,success.

-----

We have also worked bbs in full support operation with  a

total of 24 channels. We used tnc-2 on the  1st  port,pk232

on the 2nd,drsi on 5th,6th virtual ports,tnc-2  on  the  3d

port,remote qsy on the 4th port.(Qsy not under the included

driver). We had 4 channels on the 1st  port,8  channels  on

the 2nd,4 channels on 5th-6th and  8  channels  on  the  3d

port.The whole procedure ran on a PC-XT  machine   -   8MHz

clock - 640k total memory.5 days continous duty. The   only

problem we 've met was the memory  limit.We  hadn't  enough

space to work the editor with larger than 2k files  and  we

couldn't handle more than 200-300 bids.Drsi's tnctsr driver

captures about 40k,so these  problems  may  not  exist   if

instead of drsi we use some other tnc.Cache  programs  will

not allow such configuration,because of their at least  64k

memory capture.

-----

Drsi gave us no problem also.You can work this card  with

less memory allocation than the default,because it supports

only 4 channels. We suggest an amount of 215K.

----

Enjoy!!!,Spiros.