**********  FORWARD  **********

*  This BBS offers automatic fwd of mails and bulletins  to

other bbss. You have to mark bbs stations by using the 'EU'

command and put their bid flag as 'Y'. Then you have to set

up,two forward files,telling bbs what to do.

1.BBS.FWD : In this file ,there are  instructions  for  the

bbs,about how it will get the fwd station.  Each  station's

path,must start and finish with a '---'.

Example: We suppose that we have fwd paths  with  SV1IW  ON

the   1st   port   AND   SV1JO   ON   the   2nd   port.(1st

port=Com1(default 0),2nd  port=Com2(default  1).  For  DRSI

card,use always virtual ports,one more above your  existing

ports.)

The fwd file will look like this:

---

0 SV1IW N          (PORTNo CALL TYPE OF FWD)

T 0023             (TIME - MIN TIME- MAX TIME)

C SV1IW            (CMD SEND TO TNC)

---

1 SV1JO N

T 0023

C SV1JO

---

If we want to connect to SV1IW From a node ON VHF  we  have

to add:

(We suppose that node's call is J47VAA(TheNet).)

---

1 SV1IW N

T 0023

C J47VAA

S*C SV1IW            (TEXT TO SEND)

R*Connected to       (TEXT WE MUST RECEIVE IF ALL OK)

----

Another example via KANODE.

---

1 SV7QI N

T 0023

C SV7NW-2

M*?                (Wait until receive a '?')

S*C SV7QI          (Text to send)

R*###LINK MADE     (Text we must receive)

---

An example for the DRSI card.

---

2 SV7QI N

T 0023              (Default virtual port)

C SV7QI

---

2 IT9UBA N

T 0023              (2nd virtual port)

C1:IT9UBA-8

---

If in case of a port number,we add :

1) T,then export procedure starts for that station when fwd

starts.

e.g:

---

T SV7AIZ N            (EXPORT CODE,CALL,FWD TYPE(always N))

T 0023                (TIME TO DO THIS)

SV7AIZ.TXT            (TEXT FILE TO EXPORT)

---

2) D,then bbs executes dos command.

e.g

---

D SOME.COM

T 1114

---

If D becomes DN,bbs executes dos cmd only when it is free

from connections.

e.g

---

DN QSY

T 1213     (In case of at least one  connected  channel,bbs

ignores this topic.)

---

3) S,then bbs disconnects all the channels.

e.g

---

S

T 1214

---

4) L,then bbs locks.It is the same as we  send  Y0  and  MN

commands to the tncs.

e.g

---

L

T 1214

---

5) U,then bbs unlocks the channels.

e.g

---

U

T 1214

---

6) C,Then bbs sends tnc commands to the specified channel.

e.g.

---

C 3

T 0023

N15

T50

---

*  Procedures  3,4,5  are  useful  when  bbs  executes  dos

commands during fwd. If you  wish  for  example  to  run  a

housekeeping program or something else which will hold  its

process for some  time,then  you  do  not  want  to  accept

connections.

Fwd types:

N   : For normal fwd:

RO  : For reverse fwd only.

RF  : Start fwd,even if we haven't any msg to send.

** Other options. **

-  We can specify the max length of files we fwd  to  each

station by adding a '<x'   description. We  can  have  more

than one 'T' descriptions.

For example:

---

0 PA3AIR N

T 0003 <2     (From 00 to 03 anything less than 2kbytes)

T 0409 <4     (From 04 to 09 anything less than 4kbytes)

T 1023        (From 10 to 23 any length)

C PA3AIR

---

-  We can specify which days we'll fwd to each  station  by

adding a 'x-y' description.

Days are :

0:Sunday.

1:Monday.

2:Tuesday.

3:Wednesday.

4:Thursday.

5:Friday.

6:Saturday.

For example :

---

0 PA3AIR N

T  0003  0-3  (From 00 to 03 on Sunday through Wednesday)

T  0410  4-6  (From 04 to 10 on Thursday through Saturday)

T  1219  1-4  <5   (From 12 to 19 on Monday through

C PA3AIR            Thursday,less than 5kbytes)

---

-  We can fwd on more than one channels or ports at  the

same time,during each fwd cycle or during manualy  fwd.  In

the BBS.FWD file,we have to add a '>' as the  last  cmd  on

each path, we don't wish to lock.

For example, if we have to do fwd on ports 0 and 1  at  the

same time,to avoid

loss of time,we have to add.

---

0 IN3TUR N

T 0023 <5

C IN3TUR-8

>                            (Continue with the next path.)

---

1 SV7QI N

T 0023

C SV7QI                      (Don't continue,until

---                           disconnect or end of fwd.)

0 PA3AIR N

T 0023 <5

C PA3AIR

---

2.DEST.FWD : Destination file.

When we receive a bulletin or a  mail,bbs  has  to  know

where to send it. You  have  to  set  this  file  correct.A

destination description,must start and finish, also with  a

'---'. Destinations are, calls with a  '@'  in  front,which

means,bbs checks only the @bbs  field  of  the  msg,missing

calls with a '*',which means,bbs matches the  address  call

up to '*'  and  full  calls,which  means  bbs  matches  the

address call.  In  case  of  hierarchical  fwd,we  can  add

descriptions with a '.' in front.

Example :

We suppose @EU bulletins have to go  to  SV1IW,SV1JO.SV1XXX

mails have to go to SV1JO and SV1IW mails  have  to  go  to

SV1IW.

The destination file will look like this:

---

@EU

SV1IW

SV1JO

---

@SV1IW

SV1IW

---

@ALLS*

SV1IW

---

@SV1JO

SV1JO

---

SV1IW

SV1IW

---

SV1*  (every * goes last)

SV1JO

---

.NA   (Hierarchical fwd.BBs matches any NA in the @ field.)

EI6EH

5B4TX

---

.USA

4X1RU

---

OE*      (We have a different WAY here,which decreases the

DK8AT    (time of a mail to catch its destination.

IT9UBA   (We say that all OE mails

SV1IW    (have to go to DK8AT or IT9UBA  or  SV1IW.It

---      (means,this  mail  will  go  to  one   of   these

stations,except we have declared the type of msg

as 'S'.)

** MORE OPTIONS

-  We can have two wildcard callsigns,which match  any

call to a given bbs station. If we wish to send all unknown

destinations to SV7QI,we have to add the  following,as  the

last  always,description.(Use  with care)

---

#          (It works only when there is no  @bbs  field  in

SV7QI       the header)

---

$          (It works in any case)

SV7QI

---

** BULLETIN HOLD

- We can hold any bulletin,or  groups   of   bulletins

from being fwdded to  any  station.This  can  be  done,  by

creating a file  with  the name of the station we  have  to

hold.This  file,will  have   an   '.HLD'   as  extension.It

has the same format,like the 'DEST.FWD' file,  but  without

any dashes.

Let's see an example:

We wish  to  hold  all  these  bulletins  addressed  to

AMSAT,all these with  BBS  field  ALLE  ,  all  these  msgs

addressed  to  any  SV7XXX,all these come  from  I0IEX  and

all these  come  from  any  I0,from   being   forwarded  to

IN3TUR. We wish also to  hold  msg  1215  and   all   these

messages between 900 and 1000. We have  to  create  a  text

file named  'IN3TUR.HLD'  with   an  editor,  or  with  the

built in editor, and put the following in it :

@ALLE

AMSAT

SV7*

<I0IEX

<I0*

#1215

#1000 - 900

Enjoy!!!Spiros.