***********  GATEWAY/NODE  **********

There are 2 conditions to work gates successfully:

1..Tnc's must accept changing of calls between streams.

2..Tnc's must change calls even if there is  a  connected

channel. IF one of these  rules  doesn't  occur,then  don't

support gateway/node channels.They will not work.

The maximum number of gates or nodes or  both  is  always

two.One per tnc. If you have more  than  2  tnc's,then  you

have to choose only 2 of them. Definition of  gates,in  the

cnf file.Just put a 'G' in the channel's gate flag.It    is

selectable to work with both gateway/node or node only.

Gate prompt controls this selection.If you remove  the  'X'

command from there,then the channels  work  as  nodes  else

work as gateways/nodes. (Gate prompt has  declared  in  the

cnf file.)

Gates work succesfully under tnc-2 or drsi  card,but  not

under PK-232.So be carefull.

You can have gates between the same tnc  types.You  can't

put a gateway channel in the drsi card  and  another  in  a

tnc-2. If u have two tnc-2 put one gate channel on  each.If

u have a drsi card mark as gateway only one channel.It is a

double tnc,so we don't need a second gateway channel.

Do not use terminal mode on gates.It may cause troubles in

other channels.

----------