Documentation for WB6YMH/NK6K Stats package.

Version 1.1, October 1987.

This set of programs is intended to be the start of a family of

programs to gather performance statistics and other information

about the amateur radio packet system.  A complete background on

the project was provided in a paper presented at the 6th ARRL

Computer Networking Conference.  A copy of the paper is included

in this release.

The following programs are currently included:

STATS - monitors output from a KISS TNC, displays activity on

the screen and puts stats in LOG.

REPORT - reformats data from LOG file.

TOTALS - reformats data from LOG file.

PTOTALS - prints output from TOTALS program.

AVERAGE - averages data from REPORT program.

The programs are written C and compiled with Microsoft C

3.0 and 4.0.  They will be as easily transportable to other

versions of C as any other C program.

The STATS program uses two assembly language modules for an

interface to the PC's serial ports.  The SERIALIP.ASM module

provides interrupt driven serial I/O for IBM compatible COM1 and

COM2 serial ports.  The CDTMR.ASM module samples the Carrier

Detect line on the RS232 port at a 18.2 Hz rate. Obviously these

routines are very machine specific and would have to be heavily

modified or replaced when porting the STATS program to another

hardware environment.

Specifics on each program are discussed below.

STATS

----

This program reads AX.25 frames in the KISS TNC format.

Information about the frame is summed in various counters.  These

counters are output into a log file once every reporting

interval.  The reporting interval is currently set at five

minutes.   The format of the records in the log file are in

MONFILE.H

At the end of each interval, STATS opens "log" in the current

directory, appends the new records, and then closes the file.  If

the log file can not be opened (disk full), STATS aborts.

STATS also writes all incoming frames on stdout.

The STATS program may be used either standalone or with an

W0RLI/WA7MBL compatible "mbbios" program.  The standalone version

supports a standard COM1 or COM2 serial port.  Non-standard ports

are supported by use of the "mbbios" program.  STATS will use the

mbbios program instead of the internal serial routine if mbbios

is loaded.

Commands to STATS can be entered on the startup line, or after it

is running.  Once running, the command parser is entered by

hitting Alt-C.  The parser will then accept one command line,

then return to collecting data.  Multiple commands are separated

by ';'.  The commands are:

BASE

Sets the base address of the COM port. This command is used

when a "mbbios" program is used to support a non-standard COM

port.  The specified BASE address is used by the background

routine to sample the CD status line.  The BASE address is

automatically set by the PORT command when the built in

interrupt handler is used.  Example: BASE 768 sets the base

address of a non-standard serial port to 768 (300 hex).

BAUD

Selects baud rate of the serial line to the KISS TNC, e.g.

BAUD 4800.

END

Terminates execution.  Alt-X may also be used to exit

instead of Alt-C END.

HELP

Displays the list of available commands.

QUIT

Synonym for END.

PORT

Selects the port for the KISS TNC connection, n = 0 or 1 if

mbbios is not used.  Otherwise n maybe any port supported by

mbbios. The default is 0 (com1).

TIME

Adds timestamp to each frame displayed by STATS.  Timestamp

records are always included in the log file.

The following hardware mod should be made if DCD on/off times are

desired.

Since most of the current TNC designs use the DCD signal on the RS-232

interface as a connect status indicator it is necessary to modify

the TNC hardware slightly to provide a true modem DCD on the RS-232

interface.  The modification for the TNC 2 and clones is very

simple, consisting of a single jumper wire.  The jumper goes

between pin 2 of the modem disconnect header (DCD output from the

modem) and the pin of JMP1 which is NOT connected to +5 volts

(input to the DCD driver).  On the MFJ-1270B artwork the correct

pin of JMP1 is the one closest to the front panel.  The authors

have not researched modifications to other TNC designs, but it is

expected the modifications will be similar.  It is NOT necessary

to perform the DCD modification to run the monitoring software,

it is only necessary if the statistics of DCD activity are

desired.

Most terminal software used on packet will be unaffected by this

modification, however most BBS software will require the jumper to

be removed for normal operation.

REPORT

------

Report further summarizes and combines the information in the log

file built by STATS.  Data is emitted as one record per interval.

These records can then be passed through AVERAGE to produce data

averaged over larger intervals.  The data can then passed to a

plotting program for display.  Lotus 123 was used to produce the

graphs in the conference proceedings.  The various items output

by REPORT have no great cosmic meaning, they serve as examples of

the kind of information we feel can tell us something about how

the network is running.  The intent is for others to add more

sophisticated reports as we get an idea of what it is we need to

know.

The commands are summarized below.  An item enclosed in [] is

optional.

CIRCUIT

Most of the data graphed in the conference proceedings was

generated by this command.  It has no options.  The command

generates one record per interval; the records contain the

following fields:

time_stamp          Time stamp of the start of the

interval.

all circuits        Count of unique to/from pairs.

user circuits       Circuits excluding those sending

only occasional UI frames.  These

are assumed to be beacons or NETROM

node commands.

packets             All packets on channel

retried packets     Total data packets - unique data

packets

poll                All packets with poll bit set

final               All packets with final bit set

rnr                 RNR packets

rej                 REJ packets

bytes               total bytes (all bytes in all

packets)

udbytes             unique data bytes (data bytes in I

frames of unique packets)

END

Exits REPORT.

HELP

Generates a list of valid commands.

INPUT file

Selects a file to use in place of the default "log".

OUTPUT file

Selects a file to use for the generated report instead of

stdout.

HELP

Displays the list of available commands.

QUIT

Exits REPORT.

RAW  [T]

This command displays all fields of all records selected by

the RECORD command.  Hit the space key to abort the display.

The number printed under the TO call for circuit records is

the record number.  RAW also totals all the data in F records

and prints this total at the end of the display, providing a

quick indication of the amount of data collect.  This command

has one option, T.  If T is specified, only the F record

total line is displayed.

RECORD [c][d][f][i][t]

This command selects record types for processing.  The record

types are defined in the MONFILE.H file.  To select a record

type, enter the type code(s) after the command name, e.g.

RECORD FT

selects timestamp and frequency records.  Record with no

options will display the current setting.  The default is all

types.

RR

RR has no options.  The command generates one record per

interval; the records contain the following fields:

time_stamp          Time stamp of the start of the

interval.

packets             Total packets

i packets           total non-digipeated I frames

rr packets          total non-digipeater RR frames

SELECT [call]

This command selects a callsign.  Only circuit records

containing this call as one of the endpoints are used in

other processing.  The default is no call specified, i.e.,

all calls are used.  Only one call may be specified at a

time, use select with no call to de-select.

AVERAGE

-------

This program takes the output from the one-interval-per-record

output of REPORT and averages them together.  Average takes two

commands on the startup line, -n and -t.  -n is followed by the

number of records that should be averaged.  For instance, the

basic reporting interval is five minutes.  If the data is to be

plotted in half hour points, use -n6.  This will average six

five-minute intervals together.

If -t is specified, the first data item in each record is assumed

to be a timestamp, which is then not averaged.

TOTALS

------

This program reads a log file through STDIN and generals a totals

file through STDOUT.  It produces four record types, detailed

below.  This program generates one record per station on the air,

one record per digipeater, and one record giving totals of all

packets and bytes heard on the frequency.  PTOTALS is used to

display the information output by TOTALS.

TOTALS takes one option on the command line, a call sign.  If

specified, only circuit records containing the specified call as

one of the end points are used.  Record types are:

D - Totals for each digipeater.

call

total bytes

total packets

F - Totals for the frequency

total packets

total bytes

unique packets

unique bytes

l32 packets <= 32 bytes long

l64 packets <= 64 bytes long

l128 packets <= 128 bytes long

l256 packets <= 256 bytes long

g256 packets > 256 bytes long

dcd on ticks

dcd off ticks

S - totals for a station

station call

total rx bytes

total rx unique data bytes

total rx non-digipeated data bytes

total rx data bytes

total tx bytes

total tx unique data bytes

total tx non-digipeated data bytes

total tx data bytes

If a callsign was specified, a Z record is emitted as the first

record in the output file.  This record is the same as an S

record, and contains the data for the selected call.

PTOTALS

-------

This program formats the output from the TOTALS program.

There are no options.