MBBIOS is an assembler language program that captures INT 14 (the COM

service call) and extends it with support for the 8250 async port in

buffered mode and adds support for the 8530 PACCOMM PC-100 packet board.

It is configured by the file MBCNF.ASM.  MBBIOS is a resident extension

to DOS and only has to be invoked once.

PC-100 support is NOT normally included in the files.

Copyright 1986, 1987, 1988, 1989, 1990, 1991, 1992 by H. Roy Engehausen.

All rights reserved.  This software may be freely distributed and used,

but it may not under any circumstances be sold by anyone other than the

author.  It may be distributed by a commercial company as long as it is

for no cost.

================================================================================

!                                                                              !

!  Changes                                                                     !

!                                                                              !

================================================================================

V 3.5 -- 16550A support, Desqview support, improved break.

Thanks to N2GTE for the latter two items.  See notes

further down for enabling Desqview support

V 3.6 -- High speed handshaking.  Remove PC-100 support as default.

V 3.7 -- Fix bugs in handshake.  Remove use of DTR in handshake as default.

V 3.8 -- Variable handshaking.  Add control function.

================================================================================

!                                                                              !

!  File contents                                                               !

!                                                                              !

================================================================================

Files included are:

MBBIOS  .COM   -- The resident program.  Comes configured for COM1 as

standard ASYNC

MBBIOSDT.COM   -- The resident program but for units that use DTR for

handshaking.  See note below under hardware handshaking.

MBBCONFG.COM   -- Configuration control program

MS400   .MOD   -- How you share interrupts on a DFI MS-400 card

This ZIP file may contain the source code.  If not, you can get it by

sending $5 US (or equivalent) to:

Dave Larton, N6JQJ

353-C East 10th Street, Box 124

Gilroy, CA  95020-4149

(408) 778-7265

8250con .asm   -- 8250 hardware constants

8250ini .asm   -- 8250 initialization

8250int .asm   -- 8250 interrupt handler

8250svc .asm   -- 8250 service call handler

8250svcx.asm   -- 8250 service call handler (extended)

8250tmr .asm   -- 8250 timer pop handler

qrqpcon .asm   -- Quadram QuadPort hardware constants

qrqpini .asm   -- Quadram QuadPort initialization

8530con .asm   -- 8530 hardware constants

8530ini .asm   -- 8530 initialization

8530int .asm   -- 8530 interrupt handler

8530svc .asm   -- 8530 service routines

8530tmr .asm   -- 8530 timer pop handler

kiss    .asm   -- KISS constants

asmmb   .bat   -- BAT routine to assemble MBBIOS

mbbios  .asm   -- Main line

mbbcom  .asm   -- Control block format

mbbinit .asm   -- Initialization routine

mbbinter.asm   -- Interrupt handler

mbbload .asm   -- Load MBBIOS

mbbsubr .asm   -- General Subroutines

mbbtimer.asm   -- Timer pop handler

mbbusvc .asm   -- User request service routine

desqview.asm   -- Interface to desqview

mbcnf   .asm   -- ASM file configured for COM1 and two PC-100 ports

================================================================================

!                                                                              !

!  Configuring                                                                 !

!                                                                              !

================================================================================

Use MBBCONFG to configure the tables inside MBBIOS with regards to your

particular hardware configuration.  All ports which share interrupts must be

defined!  Thus if you a port on a PC-1XX, define both ports.  If you use a

port on a MS-400 with the diode modification, you must define all the ports

with diodes on them.

================================================================================

!                                                                              !

!  Parameter explanations                                                      !

!                                                                              !

================================================================================

The HIGH SPEED option is used to shift the data rate table.  See the

explanation on AH=0 below.

TRANSMIT BUFFERING allows MBBIOS to return control to the calling

program before the character is fully sent.

HARDWARE HANDSHAKING makes MBBIOS follow the RTS/CTS handshaking system

used by many TNCs.  ***NOTE*** TNC-1s are no longer supported for

handshaking.  If this is a problem, use the MBBIOSDT.COM file instead.

This does not affect TNC-1 with AA4RE BBS since that uses host mode.

Recommended settings:

Type BBS -->          RLI/MBL/BQE       Host mode (e.g. AA4RE BB)

Transmit Buffering    Try ON.  If       Must be ON

you get hangs,

OFF

Hardware Handshake    Must be ON        Must be OFF

================================================================================

!                                                                              !

!  Using                                                                       !

!                                                                              !

================================================================================

Simply execute MBBIOS before running the program that needs it.  If you

wish to unload MBBIOS for some reason, issue MBBIOS /U.

Desqview users should specify /d when loading.  Example:  MBBIOS /D

================================================================================

!                                                                              !

!  Software Notes                                                              !

!                                                                              !

================================================================================

--------------------------------------------------------------------------------

!                                                                              !

!  ASYNC + Extended Support for BBS                                            !

!                                                                              !

--------------------------------------------------------------------------------

MBBIOS supports the standard INT 14 calls plus the extended ones as

defined by Jeff Jacobsen, WA7MBL for his BBS program.  These are

explained below.  AH=0 to AH=3 are identical to the IBM BIOS definition.

AH = 0     Initialize

---------------------

This initializes the communications port.  AL contains parity, stop bit,

and word length as in the standard BIOS call.  On return, AH contains

the Line Status Register information (except bit 0 - Data ready tells

whether there is information in the extended buffer rather than the SIO

chip receive buffer).  AL contains the Modem Status Register.

The baud rate is selected via AL.  If the high speed option is off, the

rate is a specified for the standard BIOS call.  If the high speed

option is on the speed settings are from the alternate table

High speed option off:

MSB                                                     LSB

7       6       5       4       3       2       1       0

----data-rate----       -Parity--    Stopbits   --Word length--

000 -   110             X0 - none      0 - 1    10 - 7 bits

001 -   150             01 - odd       1 - 2    11 - 8 bits

010 -   300             11 - even

011 -   600

100 -  1200

101 -  2400

110 -  4800

111 -  9600

High speed option on:

MSB                                                     LSB

7       6       5       4       3       2       1       0

----data-rate----       -Parity--    Stopbits   --Word length--

000 -    9600           X0 - none      0 - 1    10 - 7 bits

001 -   14400           01 - odd       1 - 2    11 - 8 bits

010 -   19200           11 - even

011 -   28800

100 -   38400

101 -   57600

110 -  115200

111 -  330400

The high speed option can be set either by MBBCONFG or by software

using AH=9 (see below).

AH = 1     Send Character

-------------------------

AL contains the character to send.  Return values are the same as above.

AH = 2     Receive Character

----------------------------

On return, AL contains the character received.  AH contains the

composite receive LSR.  This is done by ORing all LSRs on receive

interrupts together.  Therefore the LSR returned does not necessarily

match the LSR for the the character this routine returns).  Again, AH

bit 0 indicates if there is additional characters in the buffer.  This

routine does NOT timeout while waiting for a character but will hang

indefinitely waiting for a character to appear.

AH = 3     Return Status

------------------------

Return values are the same as returned by the Initialize routine.

AH = 4     Inquiry

------------------

AH returns 0AAH.  AL returns 055H.  This call is used to see if the

MBBIOS driver has been loaded for a particular port.

AH = 5     Drop Handshaking

---------------------------

This drops the handshaking signals.  Used to tell the TNC not to

send any more information.  No return value.

AH = 6     Raise Handshaking

----------------------------

Turns the handshaking bits back on.

AH = 7     Send Break

---------------------

Generates a "Break".  Useful for getting TNC-2 or PK-232 into Command

mode from Converse or Transparent mode.  (However, if the TNC is sending

characters at the time, they will be garbled.)  No return value.

AH = 8     Non-destructive Read

-------------------------------

Returns the next character in the buffer in AL without removing it from

the buffer.  AH value defined as in AH=2 so you can use the Data Ready

bit to check to see if you really did get a character.

AH = 9     Set/Get options

--------------------------

Set the current option byte to AL returning the old option byte in AL.

The option byte is defined as follows:

.......xB    Transmit buffering (1 = on, 0 = off)

.....x..B    Hardware handshaking (1 = on, 0 = off)

..x.....B    High speed option (1 = on, 0 = off)

xx.xx.x.B    Reserved.  May be in use on other types of devices

AH = A     Write buffer

-----------------------

Sends CX characters starting at DI:ES.  Upon return, DI will be updated

and CX will contain the unsent character count.  Unsent characters could

result from either BUFFER FULL or hardware handshake is OFF.  AX will

be set as in AH=3 (Status) above.

AH = B Read buffer

------------------

Receives up to CX characters starting at DI:ES.  Upon return, CX set to

actual count read and DI is updated.  If the buffer is empty, return is

immediate with CX = 0. On return, AL contains the MSR and AH has the

composite receive LSR.  This is done by ORing all LSRs on receive

interrupts together.  Therefore the LSR returned does not necessarily

match the LSR for the the character this routine returns).  Again, AH

bit 0 indicates if there is additional characters in the buffer.  This

could happen if characters were received between the time of the buffer

movement and the loading of the LSR into AH.

AH = D Control

--------------

Subcommand AL = 0

-----------------

Gives the available bytes in the buffers.  Returns AX = bytes

available in TX buffer and CX = bytes available in RX buffer.

Subcommand AL = 1

-----------------

Turns off all modem controls (DTR RTS, etc).

Subcommand AL = 2

-----------------

Turns on all modem controls (DTR RTS, etc).

Subcommand AL = 3

-----------------

CL is used to set the handshake byte.

On exit, CL contains the previous handshake byte.

--------------------------------------------------------------------------------

!                                                                              !

!  PACCOM PC-100, PC-110, PC-120 support                                       !

!           This may not be included in this version...                        !

--------------------------------------------------------------------------------

AH = 0     Initialize

---------------------

Parms

xxx..... = Data rate.  Use same settings as IBM ROM BIOS.

300 bps = 2, 1200 bps= 4.

...x.... = Full duplex if 1.  Half duplex if 0.

....x... = Error packet handler.  0 = throw away all error

packets.

.....xxx = Don't care.  Use 0 for now!

Special case -- If AL = 0 then the COM port is closed!

The bits shown are used for parity on an ASYNC port and will be zero for

a KISS interface so it should be compatible with TCP/IP usage of INT 14.

AH = 1,2   Send/Receive Characters

----------------------------------

See above.  Packets are handled in KISS format.  Note that all commands

which set time are rounded to the nearest 50ms value.

AH = 3     Status

-----------------

Returns AH.. AL=0

0.00000. = not assigned.  Zero

.x...... = If 0 then packets remain in the transmit queue

.......x = If 1 then data await in the receive queue

These bits correspond to THRE and RDA so are compatible to ASYNC ports

AH = 4     Inquiry

------------------

AH returns 0AAH.  AL returns 055H.  This call is used to see if the

MBBIOS driver has been loaded for a particular port.

AH = 10    Send a packet

------------------------

ES = Segment of buffer containing the packet to be sent.  Note: The

buffer pointed to by ES will be freed when the packet is sent unless

this is turned off.  It should be gotten via Opcode 12. It must be 512

bytes long and conform to the layout of the buffers

Buffer Layout:

0 - 503 -- data (504 bytes long)

504 - 505 -- 16 bit data length

506       -- flags/status #1

x........ = If 1 then don't discard

this buffer after xmt.

.x....... = If 1 then this buffer has

been transmitted

..0000000 = not used

507       -- flags/status #2

000000000 = not used

508 - 509 -- User field.  Not used by MBBIOS

510 - 511 -- MBBIOS uses as a pointer to next buffer in chain

AH = 11    Receive a packet

---------------------------

Returns ES with segment of buffer containing the packet received.  If

zero, no such packet exists.

AH = 12    Buffer management

----------------------------

If ES is zero then returns a buffer segment in ES.  If ES is non-zero

then the buffer pointed to is freed.

AH = 13    Set TXD -- Time from RTS to start or packet

------------------------------------------------------

AL = new setting in clock counts (54.3 ms)

AH = 14    Set P (persistance) -- Controls time from end of DCD to RTS

----------------------------------------------------------------------

AL = new setting.  255 = 1.0

AH = 15    Set Slot time -- Controls time from end of DCD to RTS

----------------------------------------------------------------

AL = new setting in clock counts (54.3 ms)

AH = 16    Set CRC wait -- time from start of last character to drop RTS

------------------------------------------------------------------------

AL = new setting in clock counts (54.3 ms)

Should be 5 character lengths minimum