YAPP(tm) - Yet Another Packet Program

Version 2.0

(C) Copyright 1986 by Jeff Jacobsen, WA7MBL

Released For Public Distribution

ALL RIGHTS RESERVED

Permission is granted to freely distribute this code,

provided that this notice and the following disclaimer are

included in their entirety and without modifications of any

sort.  This work may not be sold, or modified and sold, or

included with any other product to be sold, without the

written permission of the author.

No fee or compensation may be requested or received by any

person or organization distributing this program, except by

the author or an authorized distributor.

Disclaimer:

No guarantees or warranties of any kind are expressed or

implied.  This code is distributed "AS IS" without any

warranty of any kind, either expressed or implied,

including, but not limited to the implied warranties of

merchantability and fitness for a particular purpose.  You

are soley responsible for the selection of the program to

achieve your intended results and for the results actually

obtained.  Should the program prove defective, you (and not

the author) assume the entire cost of all necessary

servicing, repair, or correction.

Neither the author nor anyone else who has been involved in

the creation, production or delivery of this program shall

be liable for any direct, indirect, consequential or

incidental damages arising out of the use or inability to

use this program.

Jeff Jacobsen, WA7MBL

1400 E 900 N

Logan, Utah  84321

INTRODUCTION

YAPP(tm) was written to provide a terminal program for use

in testing the WA7MBL BBS code.

A contribution of $20 is asked of any users of YAPP who

find it of value.

User feedback and registration is also encouraged.  Your

comments and suggestions are welcomed.  The success of this

method of distribution will determine whether future

versions of YAPP will be distributed freely, or sold

commercially.

Whether you make a contribution or not, you are encouraged

to copy and distribute this program for the private,

non-commercial use of others.

A copy of this program may be obtained by either sending $5

or by sending a blank, formatted disk together with an

addressed, postage-paid return mailer to:

Wes Morris, K7PYK

7422 E McKinley Street

Scottsdale, AZ  85257

A current copy should also be available for downloading

from Hamnet on Compuserve.

Contributions or user feedback may be sent to the K7PYK, or

directly to the author.

HARDWARE REQUIREMENTS

YAPP has been tested on an IBM PC, XT, AT, and a few

compatibles.  Another version of YAPP (YAPPB) is available

which uses an external resident I/O driver.  If you have an

MS-DOS computer which does not use the standard IBM

communication port locations, we may be able to help you

modify the driver to adapt YAPPB for your computer.

We recommend that your CONFIG.SYS file has a FILES

parameter of at least FILES=10.  We uses FILES=20 and

BUFFERS=20.

TNC SET UP

This program uses hardware handshaking.  Your TNC should

also be set for no parity and 8 bit words.  If you plan to

do binary file transfers, be sure that the TNC type is set

correctly.  TNC type 0 is used for testing and does not

send the commands to place the TNC into transparent mode.

TNC type 2 sends a break signal to place the TNC into

command mode when necessary, while TNC type 1 uses the

control-C method.  The most common problem is with

cabling.  Be sure that your cable connects pins 1-8 and pin

20 from you serial port to your TNC.

To enable hardware handshaking be sure the following are

set:

START 0

STOP  0

XFLOW OFF

XON   0

XOFF  0

If you are using split screen mode, you will probably want:

FLOW OFF

ECHO OFF

Other suggested parameters are included in TNC1SET.YAP &

TNC2SET.YAP which should be included on the program diskette.

COMMAND SUMMARY

The following commands are available from YAPP

F1    -  Display Help Menu

F2    -  Toggle printer on/off

F3    -  End File Capture

F4    -  Begin/Suspend File Capture

F5    -  Send text file

F7    -  Send binary file

F8    -  Receive binary file

F9    -  DOS Gateway

Alt-B    Send Break

Alt-C    Connect

Alt-D    Directory

Alt-E    Echo mode toggle

Alt-F    Change Port

Alt-G    Bell toggle

Alt-P    Strip 8th bit (parity) toggle

Alt-S    Split Screen toggle

Alt-T    Send Time & Date

Alt-W    Write Scroll buffer to disk

Alt-X    Exit

All characters that scroll off the top are saved in the

"scroll" buffer.  You may redisplay this buffer by pressing

Home, PgUp, or the UpArrow.  Home starts from the oldest

available screen in the buffer.  PgUp shows the previous

screen, and UpArrow the previous line.  Once you are

reviewing the scroll buffer, you may use PgDn or DownArrow

to move forward through the buffer.  End will exit the

review mode.

There are times when you wish you had saved something that

scrolls off the screen.  Alt-W will write the entire

contents of the scroll buffer to disk.

When in split screen mode, all received characters are

displayed on the top portion of the screen and everything

you type appears at the bottom.  Only that appearing in the

top portion will be saved in the scroll buffer.  If you

wish to print or capture everything, you should turn off

split screen mode.

You may also save information by pressing F4 to turn on

capture mode.  This will write all received characters (or

everything if you are not in split-screen mode) to a disk

file.

Pressing F4 again will suspend the capture, or pressing F3

will complete the capture.

F5 sends the contents of a file to the TNC.  You may use it

to send commands to the TNC or messages.

F7 is used to send a binary file using the YAPP binary

protocol.  The receiving station must also be using the

YAPP protocol.

F8 is used to receive a binary file.

F9 suspends the program and lets you run DOS commands.  To

return to YAPP type EXIT at the DOS prompt.  This feature

requires the file DOSPGM.COM.

YAPP features a connect directory.  To use it, edit a text

file called YAPP.DIR with the nickname you wish to use for

a station (up to 6 characters) followed by the connect

path.

For example:

Jeff   WA7MBL v WA7MXZ-1

Wes    K7PYK

BBS    WB7TRX v WA7MXZ-1,KD7YK-2

To connect, press Alt-C and you will be prompted for who

you wish to connect to.  Enter the name, and the connect

message will be sent to the TNC.

THE CONFIGURATION FILE   (YAPP.CNF)

A new feature in YAPP version 2.0 is the use of a

configuration file to set the desired parameters.

The configuration file is organized in the following

format:

1     {com port}

4800  {baud rate}

2     {TNC type}

YES   {split screen}

YES   {allow bells (^G)}

112   {status line attribute}

15    {standard attribute - keyboard input}

7     {received character attribute}

7     {help screen attribute}

7     {help screen border}

7     {transfer status attribute}

7     {transfer status border}

7     {set default window attribute}

7     {set default window border}

7     {directory window attribute}

7     {directory window border}

7     {misc window attribute}

7     {misc window border}

mon on

conok on

*** EOF  {end of commands sent to TNC on program start}

conok off

mon off

*** EOF  {end of commands sent to TNC on program end}

The first line selects the communications port.  YAPP

supports com1 or com2.

The next line is used to select the baud rate.

The next line select the type of TNC as described above

under TNC SET UP.  Valid types are 0, 1, and 2.

The next line selects Split Screen mode.  YES will start

YAPP in split screen mode.  NO will start with it off.  The

mode may be changed from within YAPP by pressing Alt-S.

The next line selects whether Bells (Cont-G) will be heard

at the terminal or not.  YES will sound the speaker when a

^G character is received.  NO will only display the

graphics character on the screen.  This mode may also be

changed from YAPP by pressing Alt-G.

The next 13 lines specify the colors to be used when

displaying the characters you type, characters received

from the TNC, on the status line, and by various windows.

In earlier versions of YAPP, this information was kept in

the file YAPP.COL and only used if you were using a color

monitor.

The values shown above are for use with a monochrome

monitor.  If you have a color monitor, you may want to try

the following values:

110   {status line attribute}

7     {standard attribute - keyboard input}

3     {received character attribute}

111   {help screen attribute}

111   {help screen border}

96    {transfer status attribute}

96    {transfer status border}

14    {set default window attribute}

5     {set default window border}

11    {directory window attribute}

6     {directory window border}

15    {misc window attribute}

4     {misc window border}

or experiment with other colors.  If you have a monochrome

monitor, you should stick with the monochrome defaults.

More information about color values is available in the

file COLORS.DOC.

The next series of lines up to the *** EOF will be sent as

commands to the TNC when you start the program.  These can

be used to change TNC parameters, load connect or beacon

message text, or for other purposes.

The final series of lines up to the last *** EOF will be

sent to the TNC when you exit the program.