A Packet Repeater Using the WA4DSY 56KBS Modem

Barry McLarnon, VE3JF

[Note: this article first appeared, in slightly different form, in

the May/June 1990 issue of the New England TCPer]

[Downloaded from hydra.carleton.ca /pub/hamradio/packet/modems 2-25-95 by WÝTN]

The Ottawa ARC Packet Group has been playing with the WA4DSY 56kbs

modem on point-to-point links for a couple of years now.  After the

novelty wore off a bit, we decided it would be much more fun to use

these modems in a high-speed CSMA MAN (Metropolitan Area Network),

and the idea of putting up a full-duplex repeater was born.  We

considered the possibility of putting together a linear

transponder, an approach espoused by Phil Karn.  This approach has

the advantage of having very low throughput delay, but after some

deliberation, we decided to go the more traditional route of using

a modem at the repeater to regenerate the bits.  The regeneration

approach provides a higher overall link margin, and avoids some of

the fussy details of the transponder design, such as the need for

AGC and excellent front-end bandpass filtering.

In some respects, the 56kbs repeater was easier to put together

than a low-speed repeater, because the WA4DSY design is an RF

modem.  There are no worries about finding radio hardware having

suitable IF bandwidth and group delay response, decent front-end

noise figure, and the like.  The receive and transmit IF circuitry

is already contained within the modem, and optimized for its use

(consider this before you complain that the DSY modem is too

expensive!).  The main RF components are standard receive and

transmit converters, available off the shelf.  The only sticky part

is the synchronization of the demodulated data to the transmit

clock, which is generated within the modulator portion of the

modem.  More about this below.

The RF part of the repeater consists of an omni receive antenna and

down-converter to 28-30 MHz, an up-converter from 28-30 MHz to the

output frequency, and an omni transmit antenna.  Since the signal

is wideband, a conventional single-antenna duplexer arrangement

would be difficult to accomplish; it is much easier to use both

separate antennas and different bands for the input and output.

You will almost certainly need a good bandpass cavity filter in the

down-converter front end, and it should also have a preamp with

reasonably low noise figure and good third-order intercept

characteristics.  Our repeater has its input at 220.55 MHz, and

output on 433.55.  We used a pair of bandpass cavities ahead of an

ARR preamp and down-converter, and on the transmit side, a barefoot

Microwave Modules transverter.

The necessary additions to the modem are contained on a small

board, the circuit for which is shown on the accompanying figure.

The main requirement in making the repeater work is to allow for

skew between the receive data, whose timing is determined by the

transmit clock at the transmitting station, and the local clock

which clocks the data into the repeater's transmit encoder.  Since

the clocks are derived from crystal oscillators, we know that they

won't differ by much; on the other hand, we don't know which will

be higher in frequency.  The repeater transmit clock will therefore

want to clock data into the encoder at a slightly slower or

slightly faster rate than it is clocked out of the decoder.  To

allow for either possibility, we use a FIFO buffer between the two

which is allowed to become half full at the beginning of a

transmission, at which point the encoder begins clocking the data

out of the buffer.  During the course of the transmission, the

buffer will slowly empty or fill up, depending on the relative

clock frequencies.  The length of the buffer must be such that the

transmission ends before underflow or overflow occurs.

In this design, a 32-bit FIFO is used.  The buffer is formed by

cascading two 40105 CMOS 16-bit FIFOs - this chip was chosen

because it is cheap and readily available, and the cascade

connection provides a convenient means of generating the "half

full" flag.  When the demodulator DCD goes high, a reset pulse is

generated which clears the FIFO chips and sets FF2, the flip-flop

which inhibits the transmit clock from clocking data out of FIFO2.

The receive clock begins clocking the receive data into FIFO1, and

it ripples through into the far end of FIFO2.  The strobed D latch

between the receive clock and the SI input acts as a clock

stretcher, to prevent clocking in data until the DIR signal

indicates the input is ready for new data.  The output latch

performs a similar function by qualifying the SO output clock with

the DOR signal.  When 16 bits have been clocked in, FIFO2 will be

full, and its DIR signal will remain low when the next data bit is

clocked in.  This condition is detected by FF1, which in turn

resets FF2 and removes the inhibit from the FIFO2 SO input.  The

data then begins flowing out of FIFO2 into the encoder.

A back-of-the-envelope calculation indicates that the 32 bit FIFO

depth should be adequate for any foreseeable transmission lengths

involving IP datagrams.  On the other hand, if you plan to

experiment with digital voice encoding or other modes involving

longer continuous transmissions, you might run into problems.

Depending on how good your crystals are (i.e., the color-burst

crystal used in transmit encoder oscillator), underflow or overflow

could occur within a few seconds after the beginning of a

transmission.  So, to be on the safe side, you might want to check

the transmit oscillators in all of the modems in your network. You

don't even need a frequency counter to do this - you can net all of

the oscillators to the one in the repeater (a trimmer would have to

be added to the encoder board to permit tweaking the oscillator

frequency).  Since they are capable of full-duplex operation, the

user stations can trim their transmit oscillators after the

repeater is installed, by looping back through the repeater.  Use a

dual-channel oscilloscope (or set up a Lissajous pattern on a

single-channel scope) to observe the receive and transmit clocks in

your modem while transmitting through the repeater, and adjust your

transmit oscillator until the two clocks are in phase.  If the

repeater or other stations on the network are subject to

significant temperature variations, the oscillator frequency change

with temperature should also be checked.  As a last resort, you can

increase the FIFO depth by adding more 40105s in cascade, or by

going to a different chip.

When a local IP switch is connected, the RTS line is used to switch

the transmit data source from the repeater receive data to the

local transmit data input.  If nothing is connected locally, the

RTS input should be tied so that the center pin of JMP1 is high

(JMP1 is provided to allow for the possible presence of the

inverter in the RS-232 level shifter, and possible use of either

sense of asserting the RTS input by the local source).  For

repeater operation, the jumper at JMP2 should be in the lower

position so that the transmit side of the modem runs continuously.

This minimizes the key-up time.  If you are running the receive

and transmit halves of the modem on the same frequency, you may

notice enough leakage into the receiver to hold the DCD on, which

of course is highly undesirable.  You may have to resort to cutting

the RF board in half to achieve sufficient isolation, so it's best

to avoid the problem by selecting a repeater output frequency which

requires a different modem transmit frequency (in our case, the

modem receives on 28.55, and transmits on 29.55 MHz).

The PTT driver for the external transmit up-converter includes a

watchdog timer which times out after about a minute of

uninterrupted transmit activity.  This driver is used instead of

the one on the modem RF board to avoid the need for modifying the

modem itself (the only mod needed is removal of the enable line

which goes to P1-1 of the decoder board).  The LEDs for DCD and PTT

make it a lot easier to see what the repeater is up to.  The RS-232

level shifter can of course be omitted in many cases.

It should be noted that the modification to allow the transmit side

of the modem to run continuously should be done in *all* of the

modems on the network, not just the one in the repeater.  When all

modems are so modified, you should be able to use a txdelay as low

as 10 ms to work through the repeater (provided that your up-

converter and/or power amp keys up quickly); otherwise, it will

have to be 17.5 ms or more.

The Ottawa 56kbs repeater has been on the air since early January

1990, and has been essentially trouble-free (after I replaced my

first FIFO design with one that worked!).  The antennas are on top

of a building, about 26 stories up.  I have no problem working

through the repeater from about 12 miles away, using 10W into an

indoor quarter-wave ground plane antenna that I made out of a BNC

connector and 5 lengths of wire.  The DCD circuit tends to be a

weak point in the DSY modem design, so we were apprehensive about

DCD falsing kicking the repeater on frequently; happily, this has

not turned out to be a problem.  Benchmark figures for FTP

performance through the repeater have been very good, only showing

a small decrease from those obtained on direct links, provided that

long frames (> 1Kbyte) are used.  The repeater has also recently

(September 1990) been connected to the Ottawa Hydra multiport

packet switch, and the interface is performing well.

For more information, contact bm@hydra.carleton.ca.

[1995 update: see the postscript files 56krptr1.ps and 56krptr2.ps

for the repeater schematic.  The Ottawa repeater is now interfaced

to a Gracilis PackeTen packet switch.]