9.6 Kbd In 15 Khz?

WÝTN Rev. 0, 10-28-94

This is a follow-up question regarding coordinating full duplex digital facilities on traditional voice repeater pairs.

Is it feasible, particularly with respect to adjacent channel interference, to operate a 9.6 KBd digital facility in a 15 Khz channel, i.e., with a G3RUH style modem?

I ask because it appears that the reported successful mingling of 9.6 Kbd digital facilities with voice repeaters on 2M were in geographical areas (or sub-bands) with 20 KHz channels.  In this part of the country the 2M repeater coordination plan uses 15 Khz channels above 145 MHz.

I believe the designers of the current 9.6 Kbd modems worked pretty hard to get the signal into a 20 Khz channel (including guard bands).  Thus, I wonder what the operational penalty is for squeezing the channel down by 25%, e.g., on S/N requirements, ability to adapt radios, etc.

Comments and suggestions based on practical experience, measurement, and/or theoretical prognostication will be greatly appreciated.

73/Rick W0TN  <rick_whiting@atk.com>

=============================================================

[Replies follow.]

We're doing it here in SoCal, and if we can get away with it on 15khz

channels, ANYONE can.

Trick is to keep the modulation narrow.  If the repeater is full

quieting, even micro-dev will work.  We use +/-3 kHz.

- Brian

------------------------------------------------------------------------

On Tue, 25 Oct 1994, Bdale Garbee wrote:

> We also (Colorado) have the brain-damaged 15khz channel spacing on 2m,

> but we don't have a 9600 baud repeater on 2m, so I haven't worried about it.

Bdale, when you start to have 9600 baud users they seem always to

want to use 2 meters. In Pheonix they put up a x1j node on 450 MHz and

asked all the tcp hams to use it and make it the gateway router for the

city. 3 guys got on. So they made a 9600 baud node wired to the other on

144.96 and now there are 20 stations using that for a router.

>

> Not interested in building one, either.  70cm is so much more pleasant to work

> with... the cans are rack-mountable, antennas have lower wind load for a

> given amount of gain, etc...

Your just getting lazy. Riding up a steep dirt path for 30 miles

with a 2 meter duplexer in your lap, that you tuned on for 5 hours in the

shack and don't want re-tuned by the road, is a real fun experiance.

>

> 73 - Bdale, N3EUA

>

73, de karl k5di

----------------------------------------------------------------------

On Tue, 25 Oct 1994, Klarsen wrote:

> 	Your just getting lazy. Riding up a steep dirt path for 30 miles

> with a 2 meter duplexer in your lap, that you tuned on for 5 hours in the

> shack and don't want re-tuned by the road, is a real fun experiance.

I've had that problem once before...  A friend of mine and I too the

cavity off our local 2 meter repeater and took it over to a technician to

realign...  It was perfect on the bench, but didn't survive the ride up

the mountain...

Luckly, we borrowed the tools from someone else and did the realignment

on the mountain...

I would hate to have that experience with a full set of duplexers...

----------------------------------------------------------------------------