DIGITAL REPEATER INFO

Compiled by WÝTN & WAÝCQG

Rev. 9, 12-22-94

: 	Now, we need a way to do bit regeneration (IE, very fast

: digipeater) without bothering routing.

: 	Two TEKK Micro's, and 1 9600bps modem will do nicely for this.

: Input frequency, output frequency, it's half duplex, and goes both

: ways.  The TEKK Micro's have a 3ms keyup to stable time.  Put this

: gadget on a nice repeater tower near you.  And you divide the network

: distance.  What you do is take the frequency of one area, and

: bit-regenerate it on another.

: 	IE, RX -> TX, and TX -> RX.  No TNC needed.  All you need the

: modem to do is regenerate the bits.  Course the frequencies have to be

: different, but so what.

: 	And we increase the chances for a collision by a little bit,

: but having this device installed fixes things a lot more.

A better solution is to actually build up a full-blown repeater.  Without

the cavities or a >lot< of antenna seperation the overload suceptable

receiver in the TEKK will get desensitized and your system will bounce up

and down as the received signal comes and goes.  We've seen this happen when

trying to use some of the old commercial mobile telephone (IMTS) radios with

their internal duplexers.  There is just not enough isolation there to

prevent desense - typically about 10 dB of it.  As for a bit-regenerator,

TAPR has a modem that has provisions for the bit-regen right on the PCB.

Exact prices are not in memory, but $70 for the modem, and $20 for the clock

and bit-regen options are about right.  We have 4 systems in the area now

using this hardware, and RF path problems aside, those systems work very

well.  One has been up for close to 2 years, with no maintainence required.

We used a GE Mastr II mobile for the 2M system and one of the 440 systems.

the other two 440 systems are using Motorola IMTS radios (Pulsar?).

: 	The only thing you will have to do is set your TXDELAY by

: approximately .0096/sec + .0006 (9600bps + 3ms*2 radios stabilize

: time) = .0102/sec.  So we'll settle for 2 hundreths of a second

: TXDELAY.  Not bad at all if you ask me.  Some of you may be asking why

: I'm using 9600bps as a value, and I figure that the difference between the

: time where the TNC get's the digital information and actually starts to

: make it analog (modulate it), that it's going to be about that long.

: (I could be wrong, but then again, I'm talking about pushing the

: envelope here.)

TXDelay through a bit-regen system will have to be a little longer, because

it has to recogize the incoming data before it can key up the transmitter.

Also, the receiving TNC has to get syncrhonized on the incoming data after

the repeater comes up and stablizes (about 7 mS on our modified commercial

rig).  Since a data randomizer is part of the modem specification, this adds

to the minimum TXDelay you can use, even on a simplex path.  Still is better

than 1200!

[Above from Internet Ham-Digital newsgroup]

----------------------------------------------------------------------

From: ron@chaos.eng.wayne.edu (Ron Atkinson  N8FOW)

Subject: Re: TAPR-2 TNC Mods; Bit

To: netsig@tcet.unt.edu

Date: Thu, 1 Sep 1994 02:40:11 -0400 (EDT)

> 2) I was told that the TAPR journals had several articles on

> 1200 baud bit regeneration mods for TAPR-2 clones. I purchased all

> of the TAPR back issues. Almost nothing regarding the subject

> was contained within them. Lot's of mention by people using

> packet rengerating repeaters (mostly 9600 baud) or advocating

> the use of them, but nothing in the way of how to mod a TAPR-2

> TNC for 1200 baud bit regeration.

The TAPR PSR  January 1992  Issue #45  has an article titled:

Modifying the Kantronics DE-1200 for Full-Duplex Operation

This is the same principle in modifying the tnc for bit regeneration. Also

ARRL/CRRL 9th Computer Networking Conference  papers (you know, the one

that people in this group say is out of publication :-)  also has an article

titled:

Digital Regenerator Modification for the TNC-2A.

This explains how to do it also.  It's only one IC chip that you can install

on the modem disconnect header and make it an instant bit regenerator for

a repeater. The 9th CNC paper uses a  4053  and the one in the TAPR PSR

uses a 74HC157.

--

Ron N8FOW

AMPRnet  : n8fow@n8fow.ampr.org

Internet : ron@chaos.eng.wayne.edu

aa011@detroit.freenet.org

#include <std_disclaimer.h>

/* all views and opinions expressed above are my own.

----------------------------------------------------------

A note for anyone building a bit regen repeater that also has a local packet

switch connected: the design in the TAPR article does it right, but the one

in the CNC paper does it wrong.  The problem is in the data selector which

determines whether the data being transmitted by the repeater comes from the

repeater or from the switch.  In the CNC design, this data selector is

controlled by the DCD signal from the repeater demod.  This means that when

the switch is transmitting, the merest flicker of DCD activity (falsing, or

maybe a station transmitting that is misconfigured or can't hear the repeater

too well) will completely mutilate the outgoing packet.  The TAPR design

correctly controls the data selector with the RTS signal from the switch.

For keying the repeater, all you need is the logical equivalent of ORing the

RTS from the switch and the DCD from the repeater demod.  The switch also

needs to see this DCD signal and be set up for half-duplex, so that it will

respect CSMA and not stomp on repeater users.

Building a 1200bps bit regen repeater out of a TNC is easy, and very

effective.  It's simply amazing how rare they are.  I suppose it's because

of the hard part: finding cavities, finding duplex frequency pairs for

packet, and convincing frequency coordination bodies that they are *needed*

for packet.

Barry

--

Barry McLarnon  VE3JF/VA3TCP   |  Internet: bm@hydra.carleton.ca

Ottawa Amateur Radio Club      |  AMPRnet:  barry@va3tcp.ampr.org

Packet Working Group           |  FreeNet:  aa187@freenet.carleton.ca

-----------------------------------------------------------------

From: Jeff King <wb8wka@wb8wka.ampr.org>

Subject: Re: TAPR-2 TNC Mods; Bit

To: netsig@tcet.unt.edu

Date: Fri, 02 Sep 94 20:20:44 UTC

Hi:

I've used the Jack Taylor TNC-2 mod for a regenerating repeater for about

2 years now in the Detroit area (441.125/446.125) and it does seem to work.

Did noticed some strangness in it, and relative to what Barry said in a

earlier message, I'll check into it. But it does work and certainly is

not expensive. Going to be doing a second one on 2 metres in the near

future so I'll see what can be done to fix the problem Barry reported (the

440 one will be going 9.6k bit regenerating)

The CNC 9th (the one I talk about above) did not have a FIFO in it. As such,

there may be some bit jitter but I've not had a problem with it.

The site the repeater is at requires can's on everything so the extra cost

is not much of a issue as contrasted with a simplex system. I believe there

is also a full duplex system in Tucson AZ as well that is 1.2kbaud.

73

Jeff WB8WKA    wb8wka@hamgate.merit.edu

-------------------------------------------------------------

From: Bdale Garbee <bdale@chunks.gag.com>

Subject: Re: TAPR-2 TNC Mods; Bit

To: "TAPR Networking Special Interest Group" <netsig@tcet.unt.edu>

Date: Fri, 2 Sep 1994 23:11:27 -0600

: The CNC 9th (the one I talk about above) did not have a FIFO in it. As such,

: there may be some bit jitter but I've not had a problem with it.

At 1200 baud, timing jitter just isn't an issue worth dealing with.  At 9600

or faster you'd like to, and the TAPR 9600 modem with bit regenerator does

the right thing.

73 - Bdale, N3EUA

-------------------------------------------------------------------

> *Message:

> From: MITCHELL.T at CCPO.KCPO

> Date: 10/3/94 2:38PM

> To: INTERNET:netsig@tapr.org at OCF_INFORM

> Subject: How do you ID a Regen Rpt?

> Contents:

> This may be trivial, but....

>

> How do you identify a regen repeater?  We are going to set a 9600 UHF

> regen system in service soon, and I was not sure how everyone else

> handles the requirments.  What about control?  Do you use a seperate

> controller?

The 2 potential ways I can see of doing this on-site are:

1) if using a TAPR modem for the regenerator, put it on a TNC-2 (or clone)

board and set the TNC to beacon every 9.5 minutes.  If you do put a nos box

on-site, set it to beacon for you.

2) construct a timer and an eprom/shift register with the proper HDLC bits

to key up, send some flags, then an ID packet.

>

> Also, we were thinking about having a NOS box attached to the TNC serial

> serial port and operate a POP Mail Server and Gateway.  Any ideas?  Should

> this be off site?

If the repeater is at a remote site, I woujld not reccomend putting a nos

box on-site.  The only potential advantage is if the remote site will have

other links co-located.  If there are no links, or the links can be done at

a users house just as well, put the nos box there, where humans can get at

it easily.  We have a multi-port switch at a local cell site, and while it

can be remote sysoped for most things, great care must be excercised to

prevent getting into a condition where a site visit is required to set

things right.  With a repeater, you have no hidden transmitters, so other

than txdelay considerations (very minor) a gateway or mailserver in no way

has to be at the repeater site, other than for linking purposes, as

mentioned.

>

> Want your input?

>

> Curiously,

>

> Tim KA5RYF

> Kansas City

>

>

>

Hope this helps.  We have 4 9600 repeaters in the area, two of which have

computers on-site.  A third is linked via a very long RS-232 cable ( 30

miles :-) care of US West Cellular to one of the above computers.  The

fourth system, and the only one on  two meters, is completely standalone on

a ~5000 ft mountain.  Good luck with your system, and if I can help, drop a

note.

73, Bob

----------------------------------------------------------------------------

|  Bob Donnell, KD7NM     bob@ethanac.kd7nm.ampr.org   rdonnell@eskimo.com |

|  Western Washington Amateur IP Address Coordinator   (206) 775-3651      |

----------------------------------------------------------------------------

In article <9410052222.AA16924@nadc.nadc.navy.mil> you wrote:

: I know what basic radios are being used for digital repeaters but what

: RF isolation is being used - cavities etc. I assume that the same requirements

: apply that are used for voice?

Yes.  RF is RF.

: Do TEKK radios work well (ok) on 440 in repeater service with cavities and

: a single antenna?

The TEKK radios exhibit relatively poor temperature stability compared to some

other "commercial" RF decks.  Not everyone cares about this as much as we do,

I realize, but at most repeater sites temp is an issue.  I've heard a mumble

or two about their behaviour in high-RF environments, but have no personal

data.  We have several TEKK radios running simplex as clients on our 9600

UHF repeater, but I've never considered them suitable for the environment on

the Peak...

Bdale

------------------------------------------------------------------------

We have a number of the TEKKs, both mini and micro, in our network.

Have not counted them recently, but the number is probably well

above a dozen now, and may be two dozen.

The majority are in the homes of hams.

Temp stability for these has not been a problem.

We've not seen problems in high RF environments, and in fact have

one micro running at a 50KW AM/150KW FM station transmitter site.

This site has a large herd of commercial radios as well, on various

freqs in the 150, 450, and 800 bands.  We DID put cavities on

this one ...

We have had some problems with the TEKK minis, and had to send them

to TEKK for repair.  Don't know the details, but suspect it was

problems from high RF environment.  Mostly we use the micros.

Our experience is that these radios are excellent low cost choice

for short point-to-point 9600 baud links.  Hang them off a kpc-9612,

put a couple kpc-9612s on a Data Engine, and you have a very interesting

six port switch.

Why are we doing this?  We're "going cellular" with many low and mid

level sites, and reserving the high sites for the long (100-200) mile

hops we require to get between popluation centers.

...  Ha

-------------------------------------------------------------------------

I've been told that the WACOM duplexers do not use Invar rods with which

to set cavity resonance with.  I have a WACOM on the SVALAN node and indeed,

when I check for power output from time-to-time, it requires retuning.

The SVALAN node is in a non-temperature controlled shed subject to temp

swings of 50 degrees or better.  The WACOM I have appears to use the

Sinclair Q circuit configuration and as such has very good rejection between

RX and TX, approximately 75 - 80 dB per section.  If your site is temperature

controlled and you get a good buy on a WACOM, then it probably will work

out satisfactorily.

I have no experience with the Teledyne, other than picking up what appeared

to be 220 MHz cavities at a recent swapmeet.  Turned out these guys are in

the 300 MHz range and my only option is to cut them down for 440.

I had some DB Products VHF cavities which were giving "tuning problems".

Since I needed a 220 duplexer for a proposed 9k6 backbone upgrade, I

decided to cut these "bad" units down for 220.  As it turned out, the

conversion ended up quite well.  I modified them for the Sinclair Q circuit

and the rejection notches are as deep as the WACOM.

Then, as sometimes happens, someone brought over an old Sinclair duplexer

which had the coax fittings mounted on a plate attached to the side of

the cans.  It appeared to be some sort of hybrid ring configuration up in

the high VHF region.  I converted this over to the Q circuit at 146 Mhz

and after a lot of fiddling with coupling loop placement, it shows good

bandpass - bandreject responses.

Based on this experience, I would rate the quality of construction between

the different brands of cavities as follows:

#1 Sinclair

#2 Teledyne (Haven't yet disassembled it, but appears to be well made)

#3 DB Products  (Cheaply constructed but works good)

#4 WACOM  (Tuning versus temperature problems)

In theory one would think a 2M duplexer cavity would work at the third

harmonic on 440.  Both N7CL and I have swept cavities resonant at 2M to

see what the 440 response would be.  I don't know what brand of cavity

Eric used but we both got similar results.  The 440 resonance point was

not a linear function of the 2M frequency.  It came out low.  In my case

it was necessary to tune the cavity to around 150 MHz to get a good bandpass

curve in the 440 range.  I didn't experiment further, but it does appear

2M cavities could be used as notch filters, bandpass filters, or a combination

of both at 440.

73 de Jack

------------------------------------------------------------------

> Now, for the new questions .....  if we want to build a digital repeater

> what to we need?  Obviously we need a basic repeater (I assume without

> any fancy control stuff like autopatch, voice ID, etc.); a TNC (does it

> neet the bit regerator); a high speed modem; duplexor; and antenna.  Can

> you make specific recommendations?  What we are really asking is "how do

> we start and what do we need?  Lets limit discussion to these areas and

> exclude antennas and duplexors since this has had a lot of verbage recently

> (but if you have some great new ideas, we are very interested in them!):

You don't need a TNC, unless you want to use it to transmit ID beacons

for the repeater, or you want to use it to control the repeater. You

should use a bit-regenerator, as there are very significant advantages

to be gained from using one (do you want the lecture on that?) and no

disadvantages that I know of, not even cost. You do need a modem, though

the modem and the bit regenerator are generally part of the same board.

Most people use the TAPR 9600 modem with the bit-regeneration and

stand-alone-clock modifications. That's really all you need outside of

the radio for a basic repeater, though you end up putting a TNC somewhere

in the system to identify the repeater, and you provide an out-of-band

control channel for the repeater. The out-of-band control channel can

be as simple as switching the repeater power using a control output of

another repeater at the same site, or you can use a TNC and transceiver

for the control link.

I have most of this stuff sitting on a shelf in El Cerrito, California,

along with a bunch of Micor and Mitrex radios, waiting until there are

enough people running 9600 to support a 9600 repeater in our area. It

sounds as if that will happen in early 1995.

Bruce Perens AB6YM

------------------------------------------------------------------------

> Now, for the new questions .....  if we want to build a digital repeater

> what to we need?  Obviously we need a basic repeater (I assume without

> any fancy control stuff like autopatch, voice ID, etc.); a TNC (does it

> neet the bit regerator); a high speed modem; duplexor; and antenna.  Can

> you make specific recommendations?  What we are really asking is "how do

> we start and what do we need?  Lets limit discussion to these areas and

> exclude antennas and duplexors since this has had a lot of verbage recently

> (but if you have some great new ideas, we are very interested in them!):

Assuming a 9600 system...

> TNC:

Not strictly necessary.  If there are no on-site links to other locations, a

stand-alone modem/bit regenerator is sufficient.  Off-site links work as

well as on-site, as all stations can hear all stations.  Avoids problems

with >more< transmitters/receivers at the repeater site.

> MODEM:

TAPR 9600 modem kit with clock and bit-regen options.

> BIT REGENERATOR:

See above

> FIRMWARE:

None needed if no TNC on-site.

> HOW TO TIE INTO EXISTING NETWORKS:

Add a TNC/Radio in a "user" configuration  at either 1) a nearby access

point ot the desired network, or 2) add a gateway station that can work into

the repeater and the desired existing network.

> CONFIGURATION AND INTERCONNECTION:

With the TAPR modem,DCD, PTT (and timeout) are handled by the modem.  A DTMF

decoder and latches can complete the control circuitry.

> REPEATER "RF" GEAR RECOMMENDATIONS:

Our group has had good sucess with GE MastrII and MVP mobile hardware.  In

particular we've found that these can be used without requiring

modifications to the IF strips for receiving, and the documentation

available to us so far has allowed what other modifications are needed to be

done to improve transmitter switching characteristics.  The isolation

between TX and RX stages is adequate in the MastrII's to use one chassis for

the complete repeater RF deck.  With a little more shoehorning we might have

gotten the modem inside the case too.

>

> 73

> Carl WA0CQG

>

73 yourself!  :)

Bob, KD7NM

----------------------------------------------------------------------------

|  Bob Donnell, KD7NM     bob@ethanac.kd7nm.ampr.org   rdonnell@eskimo.com |

|  Western Washington Amateur IP Address Coordinator   (206) 775-3651      |

----------------------------------------------------------------------------

You (ESTEY@SKYLER.MAVD.HONEYWELL.COM) write:

>Now, for the new questions .....  if we want to build a digital repeater

>what to we need?  Obviously we need a basic repeater (I assume without

>any fancy control stuff like autopatch, voice ID, etc.); a TNC (does it

>neet the bit regerator); a high speed modem; duplexor; and antenna.  Can

>you make specific recommendations?  What we are really asking is "how do

>we start and what do we need?  Lets limit discussion to these areas and

>exclude antennas and duplexors since this has had a lot of verbage recently

>(but if you have some great new ideas, we are very interested in them!):

>TNC:

>MODEM:

>BIT REGENERATOR:

>FIRMWARE:

>HOW TO TIE INTO EXISTING NETWORKS:

>CONFIGURATION AND INTERCONNECTION:

>REPEATER "RF" GEAR RECOMMENDATIONS:

Hi Carl --

I see a couple of other messages have been posted recently, but a couple of

comments from our experience with 19.2kB repeaters, and soon a 9600 baud one.

You <will> want to hook up a TNC to provide a link from the repeater channel

to a network backbone, another user freq, or whatever.  The TAPR 9600 modem

provides for this quite easily.  The best bet is probably to build the

modem into a TNC-2 box and kill two birds with one stone.

The other suggestion is that you make sure you have some RF folks in the

group (I know that's not a problem with your group, but a general comment).

Although repeaters aren't black magic, there's a big advantage to having

someone around who's dealt with intermod, desense, and all the other fun

stuff involved in repeater building.

73,

John

--

John Ackermann   AG9V

Internet:  jra@ag9v.ampr.org

Packet:    AG9V@N8ACV.#DAY.OH.US

======================================================================================

Posted : 10/28/94 00:52

To     : W0TN

From   : WA0CQG

Subject: Responses to digital rptr qu

R:941028/0015 16609@WB0GDB.#STP.MN.USA.NOAM

R:941026/2319 24745@WA0CQG.#MSP.MN.USA.NOAM

Several months ago I sent a bulletin asking about combining voice with data; and 1200 and 9600 baud sharing the same repeater with 1200 baud.  Here are the responses I received.  If you have more comments, send them along!

73

Carl [WA0CQG]

-----------------------------------------------------------------------------

djt.#tpa.fl.usa.noam (na Chuck Hast)

To: wa0cqg@wa0cqg.#msp.mn.usa.noam

Carl, I have built several of them for commercial use, and maybe I can help

you on that, the usage of both 1200 and 9k6 is not really a big problem for

the repeater, but it IS a bit problem for the USERS. If you combine both

1200 and 9k6 on the same channel then the 1k2 users and the 9k6 users have

to be able to detect the presence of the other speed, this is not such a big

deal at 1k2 but at 9k6, you get into a big problem, you are probably going

to detect the presence of 1k2 by looking at RF carrier, that is

a very slow detect and most 9k6 boxes will try to jump on channel before

the RFDCD goes true, your colision rate will be horrendous.

If it had to go that route, I would put up TWO repeaters, one for the 1k2

group, who will see the througput rise greatly, by eliminating the h

xx hidden terminals.

And another for the 9k6 group who will also see a great improvement in

throughput due to the same reason.

Tom Moulton (of ROSE fame) has written a special eprom for a digital repeater,

it is a frame regenerator and all is does is look for HDLC encoded data, it

does not care how big the frame is (well within reason!) nor what is inside

it all it looks for is HDLC encoding.

The firmware has to different TXDelay parameters to set, they are:

1. TXDelay, the normal TXDelay that is invoked when the controller has

to turn on the repeater transmitter and get the RF started.

2. RXDelay, this is a special delay that is invoked when another timer

which controls the amount of time that the repeater will stay keyed

after a frame has been sent has NOT timed out yet, if the RF carrier

(transmitter) is still on the air, RXDelay is invoked, and it will

send a very short front porch to get the distant TNC's in bit synch

then start pushing data. The long TXDelay was not needed because the

transmitter was still sending RF carrer.

The control of the data carrier entails a small modification to the TNC

so as to separate the ptt line keying from the audio control function,

but it is very simple to implement.

Tom's frame relay code is very effective and makes a digital repeater a

very efficient device, especially if you have a lot of SLOW radios trying

to do 9k6!

By way, I have measured some of the "plug and play" 9k6 radios TXD needs

and they are abisimal, the fastest being 150ms and the slower ones needed

at least 250 to 300ms!! By using Tom's code in your digital repeater you

can cut down on some of that time lost by not having to get the repeater

back into proper operation every time it needs to send a packet.

I have built several of these things, they are not a big deal to do, but

you need to use good commerical products.

If you want more info, please message me at the following INternet

address.

wchast@delphi.com

--------------------------------------------------------------------+

! Ciao de Chuck Hast, KP4DJT Packet E-mail: KP4DJT@KP4DJT.#TPA.FL.USA.NOAM !

! Amprnet: kp4djt.ampr.org [44.98.0.158],    Delphi: wchast@delphi.com     !

! X.121: 813040,   TPALAN Network Manager,   System running under OS/2 2.1 !

+--------------------------------------------------------------------------+

Message-Id: 534_kp4djt

From: kp4djt@kp4djt.#tpa.fl.usa.noam (na Chuck Hast)

To: wa0cqg@wa0cqg.#msp.mn.usa.noam

Ohh, yes to answer your question about voice, yes you can do it but the

packet stuff will drive the voice users bananas, my advice is NO WAY,

also a long voice QSO will cause timeouts if some one is doing some packeting

(like I do by connecting to a crowd app and just making a comment every once

in a while ) and some voice users come on the channel.

The implementation of a mixed system (voice/data) is not such a problem since

you can detect data and key off of the DCD (DIGITAL not analog) line, and

the voice can either use regular CD or you can use CCTS (better for this

app) to detect the presence of a legit voice user.

My recommendations in review

1. Build two boxes on two freq pairs, one for 1k2 crowd, and the other for

9k6 crowd.

2. Do NOT try to mix voice and data, the data will drive the voice nuts

and the voice users will time out the data users.

3. If you have enough packet users in your area, you will find that they

will rapidly congregate on the repeater carrying the speed they use,

4. You will find that the 9k6 group will start growing and a good rate with

a 9k6 repeater installed.

Again I hope this has been of some help.

Chuck.

-----------------------------------------------------------------------------

The answer in practical terms is no. It is not worth even trying.

voice, 1200 and 9600 stuff should/must have its own freqs unless you

and one or two other people are using it. If you have more than 3 people

in your clubs that might be on the air it would just not work.

Set up

You can use a voice repeater to pass 1200 baud stuff but you have to increase

some of the delay parameters to make it work right. The best

thing to do is to get a standard moble rig, power supply and Kantronics

KPC-3. The KPC-3 is low cost and low power. The software in the KPC-3 will

act like a node and allow you to connect to it and then on to another

station. 9600 baud takes a special radio or modified standard radio

plus a 9600 baud tnc. The tnc can cost you more than 200 bucks(ouch).

If the users can afford that you could go that way. I would put up the

1200 baud node first.

There is also a software package that you can get the runs in the

TNC-2 type TNCs like the MFJ-1270 and MFJ-1274 that can make them act

like a node and also route TCP/IP packets. I and in the process of

figuring that out right now.

If you need any more help please send me another message. I love to

talk about stuff like this.

The first of this year I set up a TCP/IP server for out club. We had

some problems getting a link to the guy that got us going so I am working on

getting something set up to someone else. To do that I must get the

software I mention above up and running. It is not to hard.

73 de Mike, ac4zq

-----------------------------------------------------------------------------

Hi Carl;

Yes - and maybe.  1200 and 9600 baud are not really compatible and

will QRM one another, but they _can_ both be run through the same

repeater.  I recommend contacting Brian Kantor for details.

As far as voice, you could probably use a PL to activate a

separate voice bypass circuit.  Voice and 1200 baud are no problem

if you simply reproduce the audio, but 9600 baud will not be passed

intact by voice circuits.

--

Mike Curtis, wd6ehr@k6ve.#soca.ca.usa.noam

Help fix a serious problem - use .noam instead of .na

===============================================================================

Here in SE PA and Southern NJ we are in the process of putting on

9600 baud repeaters using the TAPR and TEKK products. It will be started

right from the beginning with users encouraged to use low power (2-5 watts)

and high gain beams pointed at the repeaters. I am confident that it will

work but I expect minor problems.

If we do not do something to improve the user speed then we most certainly

going to lose many of them. 1200 baud (even 2400) is archaic in today's

world. When we have 14400 (56K compressed) and higher going over phone

lines. If we as amateur cannot do better we mind as well throw in the towel.

On the subject of Freq coordination, which is very hot in this area as

it is in many areas of the country (mostly urban), the assignments for

digital repeaters is fundamentally different than those for voice repeaters.

Voice repeaters are touted for their "coverage area" - the more the better.

Ususally they are put on the highest hill. Few use beams because there is

no need to. This means that a repeater pair assigned to this "wide coverage"

system has to be protected over a large area, even into other areas, unless

rarely used 'PL' frequency sharing is employed.

Digital repeaters, on the other hand, should be low profile "celled" design.

The repeaters itself (440) should probably be limited to 100 watts or less

ERP and antenna heights limited to coverage area line of sight. Users

should use low power (2-5 watts) with a beam (7-11 elements) pointed at

the repeater of interest, for an ERP of no more than 50 watts. Again

antennas should be mounted relatively low, only high enough to meet the

line of sight requirements to the repeater.

By following these guidelines most areas could reuse frequency pairs much

more conservatively than when using voice as we know it. In fact 2 or 3

pairs could cover and entire metropolitain area with many sites. It is

important that we get this point across to the anti-digital crowed. This

echos the AM/SSB days. We all know that SSB is superior for our objectives

to AM. Sure AM sounds better but that is about it. Many of the same

arguments can be made for digital. Conceivably 10 years from now all of

the repeaters will be digital (voice) highly compressed.

Doug

------------------------------------------------------------------------------------------

I am putting together a TEKK based 9600 baud repeater. Actually it is

together and I am curious what duplexors or TX/RX seperation others

are using. I had a Phelps Dodge (Celwave) 633 - which is not the best

duplexor. It has about 55dB isolation. The noise floor of the TEKK

precludes going much below -88dBm with this duplexor which is

unacceptable. What are others using/doing? We though of building a

notch or bandpass and putting it in the TX line before the power

amp. - Currently I am running a 20 Watt brick on the TEKK. Not much

difference though between using it or not with this noise floor problem.

TX is very clean otherwise and in fact the noise floor is probably normal

I just need more isolation. Got a FAX from Phelps Dodge on their other

duplexors and see a PD526 with 110-120 dB isolation, but I am sure a hefty

price tag.

Anyone have ideas on how to solve this one and not go the better duplexor

route. That is using a single antenna. Since the TEKK does not have that

great of a front-end sensitivity wise I really wanted to put a GasFet

preamp on it but that would add another 15-20dB of needed TX isolation.

120dB sure would be nice - but at that level other isolations come into

play. Even 100dB would give me the edge. A 40-50 dB notch in the

transmit leg at the RX freq might do it.

Doug

-----------------------------------------------------------------------

Hi. You have stumbled upon one of the basic reepeater problems. Once the

equipment is to the level where the transmitter carrier is low enuf at the

reciever port, that the reciever should be working, many times it does not

because of transmitter noise. The transmitter carrier is attenuated by the

duplexing network sufficiently. But transmitter noise usually requires much

more attenuation than a duplexor provides.

I solved this problem in the 56 KB full duplex link radio built here in

about a week. Here s how it was solved. Obtain from a scrap MICOR, one of

the exciter filters. These are 6 resonator filters. At a 8 Mhz split they

have about 55 dB attenuation. That is, a UHF mobile MICOR is where u will

find these exciter filters. To obtain that narrow band pass spec, the filters

are loosely coupled, and the whole filter has an attenuation of 6 dB at

pass band center. With a 5 Mhz split u may need more than one filter. And

of course, after the filter, u will need a Power Amp to bring the signal

back up. Details of the 56 KB FDX radio line up (for 430) are in the file

CELLNET.UPT which is available on the network in various places. The atten-

uation of the transmitter noise on the reciever channel is the sum of the

duplexor attenuation from TX port to RX port and the attenuation of the

noise by the above mentioned exciter filter. In our 56 KB FDX radio, the

transmitter noise was attenuated to -135 dBm on the reciever channel, with

10 watts out.

Many commercial rigs have exciter filtering built in. This is generally

a good idea, since radios operating in close proximity, as when two cars

using radios are side by side on the highway, can cause the transmitter

noise problem as well. With repeater splits, and good quality recievers,

desense from the other stations carrier is usually taken care of by the

multi resonator reciever front end filter. So, in the better commercial

radios, u should be able to find the exciter filtering u need for ur

project.

73, Don. wb9mjn%wb9mjn.ampr.org@wb9uus.bradley.edu

----------------------------------------------------------------------------

>

>

> Out west in a state

> larger than ALL of New England but having about 1 million population, it

> is far too expensive a solution! We are lucky to have simplex nodes with

> the X1J software that cover about 1/2 the state of New Mexico. This is

> not bad since there are large counties with NO hams of any kind living in

> them.

>

> 	So we don't NEED NEDA.

>

> -karl k5di

>

I mostly agree.  The same situation exists in Montana where about 10 major

mountain top simplex nodes cover the entire state.  We probably don't need

the "RULES"..  Though it might not hurt.  Most of the nodes in the state

( except mine :-) ) would benefit from parameter tweaking.  Heck - as far

as I can tell that applies almost everywhere...  Even noticed a minor

misconfiguration in the major Denver node stack since I've been visiting

the area.

It may be interesting to note that Montana's official packet organization

died of neglect this year.  The group of folks working on the UHF backbone

have consciously decided to not organize.  It would just get in the way

of progress.

In addition, I see where many many packet users give up in disgust

with the system.  It's usually a lack of information on how to use

what we have and not knowing what's there rather than true problems

with the packet network.  Maybe it's time to start an "educational"

organization.

On the subject of simplex systems; while cheap and easy, a network like

this is, how do I say it diplomatically, "throughput challenged"!!!   My

hypothesis is that if the mountain top VHF simplex sites were replaced

with UHF digitial regenerating repeaters and backyard node stacks (ala NEDA)

were used to link the repeaters and access the system - we'd have a major

increase in performance.

It wouldn't have optimum performance as a full-duplex dedicated frequency

system would have - but it would only require a single good radio and optionally

a TNC at a remote site instead of a minimum of 3 radios and 3 TNC's.  When the

extra expense, small number of supporting users (user?), hours on the road and

snowmobile maintenance is considered - I believe it's a reasonable compromise.

Besides - with only one radio you don't have to worry about desensing.  Thanks

to all who replied to my previous posting.  The consensious was unanimous for

adding cavities to the system.  Hopefully we'll dig up some to add next summer

along with separating the antennas.  For now - we're restricted to what's on

hand and what can fit in a pocket of a hikers backpack.  Following the

excellent advice to do it digital rather than analog I'm going to forget the

idea of bleeding audio between the two receivers (didn't get any hints of how

to do it anyway) and figure out how to cross connect a couple TNC's DCD

hardware.  I'll be able to try it at home first to see if it's worthwhile.

Thanks to Doug's MAXWAIT mods the need has been greatly reduced - at least

for the 2 IP'ers on channel.  Since the interference is not from other

packet users I can crank the AX25 MAXWAIT value way down (10 seconds) and

not be hogging the channel.  Sure is a lot better than watching the NOS box

count down for minutes while both the uplink and interfering channels are

completely quiet. (Tnx Doug!!!)  Still I think we'll have better throughput

by waiting for a clear spot instead of retrying often with packets that

can't possibly get through.

I'm getting way too long winded...  Must be the anticipation of too

much turkey this afternoon...  Happy Turkey-day everybody

------------------------------------------------------------------------

-> > Out west in a state

-> > larger than ALL of New England but having about 1 million

-> population, it > is far too expensive a solution! We are lucky to

-> have simplex nodes with > the X1J software that cover about 1/2 the

-> state of New Mexico. This is > not bad since there are large counties

-> with NO hams of any kind living in > them.

-> >

-> > So we don't NEED NEDA.

-> >

-> > -karl k5di

Thought I'd kick in some thoughts on this subject. We recently had an

experienced NEDA NODE_OP move into the area, N9XO, Brian McCarthy. Brian

brought to us a wealth of experience in the NEDA approach, along with a

lot of NEDA documentation. I poured over the docs. and read with envy

the extensive nature of the NEDA system. Our system here in Georgia is

crude in comparison. I think, though, that the NEDA system is a direct

reflection of both the topography and population distribution of the

area. As Karl notes, it wouldn't get it in NM and I don't think it hold

much promise in Georgia, either. It takes a homogenous spread of

population to have small areas have enough hams to put up a BBS and set

up a connection to the backbone. We don't have that arrangement here in

Georgia. The 5 county Atlanta area comprises 70% or so of the state

population and we have many, many hams here in Atlanta. BTW we have only

two (2) PBBS's to serve the entire community. (And we have a hard time

keeping those up, too!) What we need is high capacity links, because we

don't have the interest level to set up many small BBS systems.

We hope to operate a three-tier system in Atlanta: 1200 bps for the

newbies, 9600 to take up the upscale growth and be the user channel of

the future, and 56kbps as our backbone.

-> On the subject of simplex systems; while cheap and easy, a network

-> like this is, how do I say it diplomatically, "throughput

-> challenged"!!!   My hypothesis is that if the mountain top VHF

-> simplex sites were replaced with UHF digitial regenerating repeaters

-> and backyard node stacks (ala NEDA) were used to link the repeaters

-> and access the system - we'd have a major increase in performance.

Interesting comment. Nothing personal, but I seem to hear the same kind

of hogwash here also. In packet, it seems there are two schools: the

theorists who are constantly proposing the complex as a "snap" to

operate, and the rest who are left to make ANYTHING operate dependably.

A duplex digi is one complicated system; a simplex 9600 digi is

something most any packet group and get up and running. If it gets them

started and moves the group forward, then it did the job. It can then be

improved to full duplex ops. as the details are mastered. We'll probably

pursue that course.

I still marvel at the quantity of technical innovations that come out of

this hobby, (specially packet) and the extreme shortage of folks who are

willing to spend weekends getting systems to work.

I hope everybody takes this in a positive spirit; I certainly intend it

that way. I'd love to hear other packet groups feelings on these issues.

Regards,

Steve Diggs

East Atlanta LAN

----

Top Of The Rock BBS - Lilburn, GA         SYSOP: Steve Diggs

UUCP: totrbbs.atl.ga.us               Snailmail: 4181 Wash Lee Ct.

Phone: +1 404 921 8687                           Lilburn, GA 30247-7407

-----------------------------------------------------------------------------

From:	SMTP%"netsig@tapr.org"  7-DEC-1994 03:03:14.46

To:	ESTEY

CC:

Subj:	[NETSIG:407] Re: info/question on duplexors.

Date: Wed, 7 Dec 94 02:55 CST

Message-Id: <15@klaatu.kd7nm.ampr.org>

Reply-To: netsig@tapr.org

Originator: netsig@tapr.org

Sender: netsig@tapr.org

Precedence: bulk

From: bob@klaatu.kd7nm.ampr.org (Bob Donnell - KD7NM)

To: netsig@tapr.org

Subject: [NETSIG:407] Re: info/question on duplexors.

X-Listprocessor-Version: 6.0c -- ListProcessor by Anastasios Kotsikonas

X-Comment: Tucson Amateur Packet Radio  Networking SIG

Steve Diggs:

Here in the Seattle area we have 5 packet repeaters currently.  One is on

220 and runs at 1200 baud.  One is on 2 meters and is 9600 baud, and the

rest are on 440 at 9600 baud.  All systems are using a single antenna,

with standard cavity duplexers.  The 440 systems originally started off

using the mobile duplexers that were part of the IMTS radios that were

converted into the repeaters, but there was a lot of desense - 10 dB or more.

The other systems were built up with repeater-grade duplexers from day one.

We have used single-antenna systems because in our observations of split-

antenna repeaters (even at the same site) that the hot spots for TX and

RX are different - and more so than with single-antenna repeaters.  So we

choose (or our landlord does, in some cases) what seems to work best.

Our two-meter repeater is a GE MASTR II mobile radio - 60 watts, turned

down to about 30 watts.  Ours is a wide-band PLL-based exciter.  With

some of the mods developed by the construction team, the repeater comes

on the air within 10 mS of the modem's DCD signal coming true.  That

repeater has been on the air for 2 years now, with the only problem being

that the receiver shifts off frequency in very cold weather (like the

last few days here) unless we fire enough packets at it to warm it up.

Unfortunatly we missed our chance this fall to get the properly temp.

compensated crystals installed.  :(

Currently the 3 440 repeaters are tied together via data links provided

by US West Cellular and their ham club, and the repeaters (coincidently)

are located at cell sites with antennas on the cell towers.  As more

users get on the repeaters, along with their previous lan connections,

we are supplementing the hard-wire connections with gateways at users'

homes to tie all the repeaters together.  The concept really does work.

Hope this gives you some insights.

73, Bob KD7NM

---------------------------------------------------------------------------

|   Bob Donnell, KD7NM       bob@kd7nm.ampr.org    rdonnell@eskimo.com    |

|   Western Washington Amateur IP Address Coordinator   (206) 775-3651    |

---------------------------------------------------------------------------

===========================================================================

[12-22-94]

>I know there will probably be idealogical arguments against bridges but

>they are a useful tool and might provide some of the functionality we

>require! :-)

I considered them a long time ago but concluded that although bridging

is a very useful tool in a LAN environment, they have serious problems

in a random-access RF environment that make them less than workable.

The main problem is that bridges assume full bidirectional

connectivity within each of their subnets. That assumption is inherent

in the way a bridge works; if it hears a packet from an interface

that's destined to another host on the same side, then it takes no

action because it assumes that all hosts on that subnet can hear each

other directly. This assumption clearly doesn't work for simplex

multiple access radio.

Now if you wanted to establish bridges between full duplex repeaters

they would probably work well.  But I'm kind of down on repeaters

because of their low spectral efficiency (in terms of bits/sec/hz/sq

km).

Phil

-------------------------------------------------------------------------